# CHAPTER 10	COMMUNICATION INTERFACE§§

## 1.10	REGISTERS

### 1.1.1	DSP Registers

All synthesizer and digital audio communication is handled via four contiguous PC port addressesXE "Port Address"§ that access the DSP.  These port addresses are used to read and write 16-bit (word) data values.  The base address for the Aria system is jumper selectable between 280h, 290h (default), 2A0h, and 2B0h.  A software routine is provided that demonstrates a technique for identifying the base address.

| Port Address | READ: | WRITE: |
| --- | --- | --- |
| Base + 0 | DSP 16-BIT DATA | DSP 16-BIT DATA |
| Base + 2 | STATUS | CONTROL |
| Base + 4 |  | DMA ADDRESS |
| Base + 6 | DMA DATA | DMA DATA |

READ Port Base+0  (DSP 16-BIT DATA)

This port is used to read 16-bit data from the DSP.  The data identifies the interrupt source when an interrupt on the PC is generated by the DSP.  If the word read at this port is 0001h (bit 0 set), the interrupt has been generated by the Aria PCM digital audio task.

WRITE Port Base+0  (DSP 16-BIT DATA)

This port is used to write command sequences for control of all DSP software-based functions (See Section 3.5 DSP COMMANDS, for more information).

READ Port Base+2  (STATUS)

The 16-bit word read from this Status port is identical to the CONTROL word written to this port address.  It will reflect the same values except for the following bits:

Bit 10:	(0)  DMA transfer complete
(1)  DMA transfer incomplete

Bit 15:	(0)  DSP 16-BIT DATA port ready
(1)  DSP 16-BIT DATA port busy

WRITE Port Base+2  (CONTROL)

Each bit of the 16-bit word written to this control port has the following effect:

| Bit 0: 1: 2: 3: 4: 5-6: | Interrupt PC after a DSP Data read - set to 0 (no) Interrupt PC after a DSP Data write (0) Sound Blaster emulation (1) Aria SynthesizerXE "Aria Synthesizer"§ / PCM audio Interrupt DSP after a PC Data read - set to 0 (no) Interrupt DSP after a PC Data write (0) Sound Blaster emulation (1) Aria Synthesizer / PCM audio Interrupt PC on DMAXE "DMA"§ terminal count (0) No PC interrupt on DMA completion (1) PC interrupt generated on DMA completion Select sample rate |  |
| --- | --- | --- |
|  | BITS: 6 5 | SAMPLE RATEXE "Sample Rates"§: |
|  | 0 0 | 44.10 kHz |
|  | 0 1 | 31.50 kHz |
|  | 1 0 | 22.05 kHz - Default (use this for Aria Synthesizer) |
|  | 1 1 | 15.75 kHz |

WRITE Port Base+2  (CONTROL, continued)

| Bit 7: 8: 9: 10: 11-15: | Select synthesis method (0) Sound Blaster emulation (1) Aria Synthesizer / PCM audio Place DSP in RESET state DMA direction (0) PC to DSP transfer (1) DSP to PC transfer Initiate DMA transfer Reserved (set to zero) |
| --- | --- |

NOTE:  All bits default to (0) unless otherwise specified

WRITE Port Base+4  (DMA ADDRESS)XE "DMA"§

When starting a DMA or Direct I/O transfer between the PC and DSP, the DSP RAM address to read or write must be specified at this port.  The DSP word address will automatically increment during a transfer (see Section 3.4, DIGITAL AUDIO DATA TRANSFER).

READ/WRITE Port Base+6  (DMA DATA)XE "DMA"§

Single word data transfer can be performed by reading or writing to this port address.  The DMA ADDRESS port must first be set to an appropriate DSP RAM address.

XE "DSP Registers" \r "DSPRegisters"§

### 2.1.2	Sound Blaster Registers

Five port addresses are accessible for programming the Sound Blaster emulationXE "Sound Blaster Emulation"§ functions.  The Sound Blaster ports can read and write 8-bit (byte) data only.  The base port address is selectable between 220h and 240h.  Synthesis ports can also be referenced at addresses 388h and 389h (same as the AdLib Music Synthesizer Card).

| Port Address | READ: | WRITE: |
| --- | --- | --- |
| Base + 6 |  | RESET |
| Base + 8 or 388h | SYNTHESIS STATUS | SYNTHESIS REGISTER SELECT |
| Base + 9 or 389h |  | SYNTHESIS DATA |
| Base + A | READ DATA |  |
| Base + C | BUFFER STATUS | DATA/COMMAND |
| Base + E | DATA AVAILABLE STATUS |  |

WRITE Port Base+6  (RESET)

A write of a "1" to this port, followed by a write of a "0" will cause the Sound Blaster emulation code to initialize.  The PC must wait at least 3 microseconds between writing the "1" and writing the "0".

After the RESET has been initiated, the system will indicate DATA AVAILABLE (Port Base+E) and place the value 0AAh at the READ DATA port (Port Base+A).  It will usually take approximately 100 microseconds before this return value is available.

READ Port Base+8 or 388h  (SYNTHESIS STATUS)

This status register indicates IRQ and Timer flags as follows:

Bit 5:	T2	When timer 2 is operational, this bit is set on overflow condition (up to 320 microsecond resolution)

Bit 6:	T1	similar to timer 2 (up to 80 microsecond resolution)

Bit 7:	IRQ status

WRITE Port Base+8 or 388h  (SYNTHESIS REGISTER SELECT)

This port is used to select an address in the Register Address Map of the Sound Blaster/AdLib synthesizer section.  After every REGISTER SELECT write, the PC software must wait a minimum of 3.3 microseconds before writing the data to the SYNTHESIS DATA port.

WRITE Port Base+9 or 389h  (SYNTHESIS DATA)

After setting the SYNTHESIS REGISTER SELECT (Port Base+9) to an appropriate address, a data byte may be written to this port.  After every SYNTHESIS DATA write, the PC software must wait a minimum of 23 microseconds before writing to any other synthesis port.

READ Port Base+A  (READ DATA)

This port is used by the PC to retrieve data bytes sampled by the audio system (Sound Blaster DIRECT 8-BIT ADC mode).

READ Port Base+C  (BUFFER STATUS)

When writing commands and data to the audio system, this port indicates when the command buffer is ready to receive a new data byte.  If the most significant bit of this port is set to "1", the buffer is not ready.  If the bit is clear "0", the next command or data byte may be written.

WRITE Port Base+C  (DATA/COMMAND)

This port is used by the PC to send commands and data to the Sound Blaster emulation routine.

Supported Sound Blaster command functions are as follows:

10h	DIRECT 8-BIT PCM DATA WRITE
14h	DMA 8-BIT PCM DATA WRITE
20h	DIRECT 8-BIT PCM DATA READ
24h	DMA 8-BIT PCM DATA READ
3Ah	VOLUME SETTINGS:

Sierra-specific extended command -- Two bytes of volume follow the command, the first volume setting is the Sound Blaster PCM volume (0 to 63) and the second volume setting is the Sound Blaster FM music volume (0 to 63).  NOTE:  This command only works on systems which include Aria system ROM part SC18055/6/7/8/9.

40h	SET PCM TIME CONSTANT
80h	PLAY SILENCE
D0h	HALT DMA
D1h	TURN ON SPEAKER
D3h	TURN OFF SPEAKER
D4h	CONTINUE DMA

E0h	INVERT DATA (Data is XOR'd to DATA PORT)
E1h	RETURN VERSION CODE
F2h	GENERATE INTERRUPT

Refer to Creative Lab's Sound Blaster Developer Kit for programming details.

READ Port Base+E  (DATA AVAILABLE STATUS)XE "Sound Blaster Emulation"§

When reading data from the audio system, the most significant bit of this byte will be set to a "1", indicating data ready.  This port is also read by the PC within an interrupt service routine to acknowledge a DMA interrupt from the audio system.

XE "Sound Blaster Registers" \r "SBRegisters"§

### 2.1.3	MIDI Registers

The base address for the Aria MIDI XE "MIDI"§port can be selected between 320h and 330h.  Two 
8-bit (byte) ports are available.

| Port Address | READ: | WRITE: |
| --- | --- | --- |
| Base + 0 | MIDI DATA | MIDI DATA |
| Base + 1 | STATUS | COMMAND |

READ Port 330h  (MIDI DATA)

This port is used by the PC to retrieve the next MIDI data byte from the MIDI input interface.  STATUS port bit 7 will be zero if data is ready to be read from the MIDI DATA port.

WRITE Port 330h  (MIDI DATA)

This port is used by the PC to send both command data and MIDI data bytes to the MIDI output interface.  STATUS port bit 6 will be zero when values can be written to the DATA port.

READ Port 331h  (STATUS)

The STATUS port is used to determine if the MIDI interface is ready to send or receive data at the MIDI DATA port.  The following bits are defined:

Bit 6:	Write MIDI data status

(0)	Data can be written to MIDI DATA port

(1)	MIDI DATA port busy

Bit 7:	Read MIDI data status

(0)	Data can be read from MIDI DATA port

(1)	No data available at MIDI DATA port

When this bit changes to zero, an interrupt is
generated on the PC.

WRITE Port 331h  (COMMAND)

The COMMAND port is used to send a command to the MIDI interface.  STATUS port bit 6 will be zero when values can be written to the COMMAND port.  

The MIDI DATA port will return an acknowledge (FEh) after writing a value to the COMMAND port, except after a RESET command (FFh) from UART mode.  Any subsequent data bytes associated with a command must be written to the MIDI DATA port.

Supported MPU-401 commandsXE "MIDI:Port commands"§ are:

11h	STOP RECORD - stops time-stamped recording.  The MIDI message for stop (FCh) is not sent out by this command.

22h	START RECORD - starts time-stamped recording.  All commands received from the MIDI interface are filtered and preceded with a timing byte.  The value of the timing byte is the difference in time from the last message received, in units determined by TEMPO (E0h) and TIMEBASE (C2h - C8h) settings.  A timing byte of F8h is sent if the record counter overflows -- this value should be interpreted as 240 decimal.  The MIDI message for start (FAh) is not sent out by this command.

34h	If in 'Data In Stop' mode -- sends a zero timing byte ahead of data.

3Fh	Switch to UARTXE "UART"§ mode.  All MIDI data is passed unaltered in both directions.  All commands except RESET (FFh) are ignored.

8Ah	Data In Stop mode OFF.

8Bh	Data In Stop mode ON.

ABh	Read and clear record counter.

ACh	Read software version (returns $15).

ADh	Read software revision (returns $01).

AFh	Read TEMPO setting.

BAh	Clear record counter.

C2h - C8h	Set TIMEBASE in 'ticks per beat':

Command	C2h	C3h	C4h	C5h	C6h	C7h	C8h

Timebase	 48	 72	 96	120	144	168	192

Max Tempo	240	240	240	240	208	179	179

Min Tempo	 32	 16	 16	   8	   8	   8	   8

D0h	Want To Send Data (WSD).

D7h	Want To Send System message.

E0h	Set TEMPO in 'beats per minute (8 to 240)

FFh	RESET interface -- all parameters are set to power on defaults.

Major differences between Aria MIDIXE "MIDI"§ and Roland MPU-401 MIDI areXE "MIDI:MPU-401 differences"§XE "MPU-401"§:

·	THRU data is not supported.

·	Channel reference tables are not used.

·	There are no selectable filters on MIDI data input -- all commands except F8h, FEh and FFh are passed from MIDI input to host while in record mode.

·	Play tracks are not supported, all output data is sent using 'Want To Send Data/System' commands, with track information ignored.

·	Metronome is not supported.

·	No MIDI real-time messages are sent to the MIDI OUT port, unless transmitted by the 'Want To Send System' command.

·	Conductor mode is not supported.

For more information, consult the Roland MIDI Processing Unit MPU-401 Technical Reference Manual.XE "MIDI Registers" \r "MIDIRegisters"§

### 2.1.4	Joystick Registers

The joystickXE "Joystick"§XE "Joystick Registers"§ or game portXE "Game Adaptor"§ registers conform to the standard IBM game port specification.  Any 8-bit port address between 200h and 207h inclusive may be used for resetting and converting dual joystick values.

Any 8-bit value may be written to one of the port addresses to start the joystick analog to digital conversion.  Byte values read from the port will be as follows:

Ports 200h - 207h

| B2 | B2' | B1 | B1' | Y2 | X2 | Y1 | X1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |

B1 and B1' are bits indicating individual button/fire presses for joystick 1.  When a joystick button is pressed, the corresponding bit is set.  B2 and B2' are bits that indicate individual button/fire presses for joystick 2.

X1 and Y1 are the trigger comparator bits to calculate X and Y axis values for joystick 1.  X2 and Y2 are the trigger comparator bits to calculate X and Y axis values for joystick 2.

Each comparator bit will toggle from zero to one after some elapsed time indicating the position of the joystick.  Application software should calibrate on startup by having the user center their joystick and then calculate the time for each axis.

## 2.20	MEMORY

### 2.2.1	System ROM Configurations

Currently, the Aria system has 10 different ROM configurations.  Most are very similar to one another and differ only in the number of Aria Synthesizer wave tables.  The table below shows all ROM part numbers and the difference in features.

| ROM SIZE | ARIA BASIC | BASIC + ARIA LISTENER | BASIC + SC18075 MIXER | BASIC + MIXER + LISTENER |
| --- | --- | --- | --- | --- |
| 64 Kb | SC18050* | --- | SC18055* | --- |
| 512 Kb | SC18051 | SC18053 | SC18056 | SC18058 |
| 1 Mb | SC18052 | SC18054 | SC18057 | SC18059 |

*No Aria Synthesizer

Aria Basic includes Sound Blaster emulation, Aria PCM audio, and contained in 512 Kb and larger ROMs, the Aria Synthesizer.  Aria ListenerÔ XE "Aria Listener"§XE "Speech Recognition"§is a speaker-independent voice recognition system.  The SC18075 Mixer is Sierra's integrated mixer chip.

Each ROM part may be distinguished by a part code that can be read back from the DSP.  Memory location 6104h in DSP RAM contains the ROM part code (see Section 3.2.2 of this chapter for information on how to read DSP RAM data).  Below are the corresponding part codes and their decimal equivalents for each ROM.

| SIERRA PART # | PART CODE (HEX) | PART CODE (DECIMAL) |
| --- | --- | --- |
| SC18050 | 0x0214 | 2.20 |
| SC18051 | 0x0215 | 2.21 |
| SC18052 | 0x0216 | 2.22 |
| SC18053 | 0x021F | 2.31 |
| SC18054 | 0x0220 | 2.32 |
| SC18055 | 0x0228 | 2.40 |
| SC18056 | 0x0229 | 2.41 |
| SC18057 | 0x022A | 2.42 |
| SC18058 | 0x0233 | 2.51 |
| SC18059 | 0x0234 | 2.52 |

The "part code (decimal)" column assumes that the high-byte of the part code is a major version number and the low-byte is a minor version number.  The tenths place of the number indicates the feature set and the hundredths place indicates the ROM size.

The following is a C language example for reading the ROM part code:

int part, majorver, minorver;

outpw (baseport+4, 0x6104);	//ADDRESS Port

part = inpw (baseport+6);	//DATA Port

majorver = version >> 8;	//High byte

minorver = version & 0x00FF;	//Low byte

The equivalent assembly language code is as follows:

MOV	DX, baseport	;Setup base I/O address
ADD	DX, 4	;DSP's DMA ADDRESS Port
MOV	AX, 6104h	;Requested location
OUT	DX, AX	;Make request
ADD	DX, 2	;DSP's DMA DATA Port
IN	AX, DX	;Read data

Register AH (Most Significant Byte of AX) will contain the part's major version number and register AL (Least Significant Byte of AX) will contain the part's minor version number.  For example, the SC18051 ROM (Part Code 2.21) would return 0215h in register AX.

Please note that the feature set and ROM size information is more easily derived by reading other DSP RAM locations, which are described in the next section.XE "ROM Configuration" \r "ROMConfig"§XE "Memory:ROM Configuration" \r "ROMConfig"§

XE "ROM Part Code" \r "ROMPartCode"§

### 2.2.2	Memory Layout

Random Access Memory

The DSP RAM contains several memory locations that can be read by the PC in order to determine configuration and status:

| Word Address | Description |
| --- | --- |
| 6050h | Hardware ConfigurationXE "Hardware Configuration"§ flag: Bits 0-3: ROM Size 0000 = 64K 0001 = 128K 0010 = 256K 0011 = 512K 0100 = 1M 0101 = 2M 0110 = 4M 0111 = 8M 1000 = 16M 4: ROM Width 0 = 8 bits 1 = 16 bits 5-7: Mixer TypeXE "Mixer"§ 000 = No mixer 001 = Discrete mixer 1 100 = Integrated mixer chip (SC18075) 8-10: RAM Size 000 = 8K 001 = 16K 010 = 32K 011 = 40K 100 = 64K 11-12: ADC ResolutionXE "ADC"§ 00 = 8 bits 01 = 10 bits 10 = 12 bits 11 = 16 bits 13: Telephone Interface 0 = No telephone interface 1 = Telephone interface present 14-15: Reserved |

| Word Address | Description |
| --- | --- |
| 6052h 60C0h 60C3h 60C4h 6100h 6101h 6102h 6104h | DSP Software ConfigurationXE "Software Configuration"§ flag: Bit 0: Reserved 1: Sound Blaster emulation 2: Aria PCM digital audio 3: Aria SynthesizerXE "Aria Synthesizer"§ 4: Telephone interface 5: Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§ 6: Unused 7: ReverbXE "Reverb"§ effects 8: QSoundÔ Virtual Audio effectsXE "QSound Virtual Audio"§ DSP Operating System Version NumberXE "DSP Version Number"§: MSB = Major version number LSB = Minor version number DSP Part CodeXE "DSP Part Code"§: 0 = SC18025 DSP 1 = SC18026 DSP Reserved FIFO Buffer offset for playbackXE "FIFO Buffers"§: Toggles between 0 and packetsize during digital audio playback FIFO Buffer offset for recordingXE "FIFO Buffers"§: Toggles between 0 and packetsize during digital audio recording DSP task initialization flag: 0 = DSP task not initialized 1 = DSP task initialized ROM Part CodeXE "ROM Part Code"§: MSB = Major version number LSB = Minor version number |

These locations contain 16-bit values that can be obtained by setting the DSP DMA ADDRESS Port to the requested address and then reading the DMA DATA Port.  The following is a C language example:

int value, address=0x6102;

outpw (baseport+4, address);	//ADDRESS Port

value = inpw (baseport+6);	//DATA Port XE "Memory:RAM Locations" \r "RAMConfig"§XE "RAM Locations" \r "RAMConfig"§

Read Only MemoryXE "Memory:ROM Configuration"§XE "ROM Configuration"§

The 512 Kb and 1 Mb ROM are options on the Aria system and are mainly used for storage of synthesis wave tables.  A simple melodic wave table is usually 256 or 512 bytes in length.  Complex melodic and percussive waveforms can be any multiple of 256 bytes in length, up to a maximum of 64 Kbytes.

Up to 16 Mbytes of ROM can potentially be read by the DSP.  All Aria systems map the ROM wave data as shown below:

Byte

Offset

| 0 | ROM | \ 512 Kbytes (Wave Blocks 0 to 2047) / |
| --- | --- | --- |
| 524288 | ROM | \ 512 Kbytes (Wave Blocks 2048 to 4095) / |
| 1048576 |  |  |
|  | " | Up to 16 Mbyte total addressing space |

One "Wave Block" is always 256 bytes in length

Appendix B, "ROM WAVE LIBRARIES," contains the wave table mapping for the 
512 Kb and 1 Mb ROM options.

## 2.30	MUSIC SYNTHESIS

The Aria system is capable of producing multi-timbral synthesized music.  Two modes of operation are available - Aria Synthesizer and Sound Blaster Emulation.

### 2.3.1	Aria Synthesizer

Initialization

To initialize for Aria Wave Synthesis mode, write 00CAh to the DSP CONTROL Port.  This value must be written during initialization to switch the system from Sound Blaster emulation (default).

Next, send Command ID 00h (SYSTEM INITIALIZATION), found in Section 3.5.1, System Configuration Commands.  The command parameters must load the ROM-based application, Aria PCM Audio and Aria Synthesizer.

The system will take approximately 5 milliseconds to initialize.  To detect when the DSP is ready, DSP memory location 6102h can be polled by the PC.  The value will be set to 0001h when the system is fully initialized.  The following is a C language example:

int ready=0;

outpw (baseport+4, 0x6102);	// First, zero the flag

outpw (baseport+6, 0);

// <--- SEND INITIALIZATION COMMAND STRING HERE --->

while (ready != 1) 
	{

// NOTE: A time-out should be added!

outpw (baseport+4, 0x6102);	//ADDRESS Port

ready = inpw (baseport+6);	//DATA Port

}

The following is the equivalent assembly language code:

MOV	DX, baseport	;Setup base I/O address

ADD	DX, 4	;DSP's DMA ADDRESS Port

MOV	AX, 6102h	;Requested location

OUT	DX, AX	;Set address

ADD	DX, 2	;DSP's DMA DATA Port

XOR	AX, AX	;Write a zero

OUT	DX, AX

;; <--- SEND INITIALIZATION COMMAND STRING HERE --->

LOOP1:		;NOTE: A time-out should be added!

MOV	DX, baseport	;Setup base I/O address
	ADD	DX, 4	;DSP's DMA ADDRESS Port
	MOV	AX, 6102h	;Requested location
	OUT	DX, AX	;Make request
	ADD	DX, 2	;DSP's DMA DATA Port
	IN	AX, DX	;Read data

CMP	AX, 1	;Is value = 1?

JNE	LOOP1	;No, try again

When exiting an Aria application, be sure to set the system back to Sound Blaster mode by sending a SYSTEM INITIALIZATION command and writing 0040h to the DSP CONTROL Port.XE "System Initialization" \r "SystemInit"§

Synthesis Structures

A musical note for a particular instrument or "patch" is produced by activating a number of synthesis operators.  Voices are generated by the DSP when it sums the waveform output of each active operator.  Patches and operators are defined in a Patch Bank file (see Appendix A, "PATCH BANK FORMAT").

µ §

Figure 2.1  SYNTHESIS STRUCTURES

As shown in Figure 2.1, a musical note requires that a number of operators be activated.  It is up to a PC sound driver to determine which DSP operators are to be allocated for this purpose and then issue the OPERATOR ON commands.  Up to 32 operators are available in the system, depending upon the selected playback mode.  Each patch can define up to 8 simultaneous operators to be triggered with each note.

Each operator further defines Wave Block pointers and a Wave Parameter Table.  A Wave BlockXE "Wave Block"§ is a 256 byte section of waveform data stored in ROM.  The Wave Parameter TableXE "Wave Parameter Table"§ specifies an amplitude envelopeXE "Envelope"§ and any low frequency oscillators (LFOs) to be applied to the waveformXE "Low Frequency Oscillator"§.  A maximum of 80 Wave Parameter Tables and 16 LFO definitions may be stored and accessed by the DSP.

All Aria Synthesizer commands can be found in Section 3.5.3.  Using the synthesis commands, each operator may be individually adjusted in pitch, volume and stereo pan positioning.  Overall synthesis output volume may also be adjusted.

Playing Musical Notes

The low-level procedure for playing a musical note is as follows:

1)	Make sure the system is switched to Aria Synthesizer mode (see Command ID 00h, SYSTEM INITIALIZATION).

2)	Download one Wave Parameter Table for each operator defined by a selected instrument patch (see Command ID 02h, DOWNLOAD WAVE PARAMETER TABLE).

3)	Turn on any required LFO's as specified by the instrument patch, using the LFO ACTIVATE command (Command ID 24h).

4)	Send an OPERATOR ON command (Command ID 20h or Command ID 23h) for each operator that is to play.

5)	When the note is to be turned off, send an OPERATOR OFF command (Command ID 21h) for each operator that was turned on.

6)	Repeat steps 4) and 5) for each note that is to be played.

The high-level procedure for playing a musical note is to send MIDI commands via the MIDImessage() routine, found in the DOS APPLICATION INTERFACE Library (Chapter 6).  This routine handles all of the low-level procedures listed above.XE "Aria Synthesizer" \r "AriaSynthesizer"§XE "Aria Wave Synthesis" \r "AriaSynthesizer"§XE "Wave Synthesis" \r "AriaSynthesizer"§

### 2.3.2	Sound Blaster Emulation

On power up, the audio system defaults to Sound Blaster EmulationXE "Sound Blaster Emulation"§ mode.  This mode can also be selected by sending Command ID 00h (SYSTEM INITIALIZATION), followed by writing 0040h to the DSP CONTROL Port.

Sound Blaster mode can also be selected by writing 0140h followed by 0040h to the DSP CONTROL Port.  The system will take approximately 25 milliseconds to initialize for Sound Blaster emulation.

All Sound Blaster digital audio functions and most music synthesis functions are emulated in this mode.  Detailed programming information can be found in Creative Lab's Sound Blaster Developer Kit.

Hardware-based digital audio compression, AdLib Composite Speech Mode (CSM) and Sound Blaster MIDI are not currently supported in this emulation mode.

XE "Music Synthesis" \r "MusicSynthesis"§

## 2.40	DIGITAL AUDIO

The Aria system is capable of transferring digital audio data to and from the PC using two different methods.

The first data transfer method uses a Direct Input and Output XE "Direct Input and Output"§double buffering scheme.  The other method utilizes Direct Memory Access (DMA) XE "DMA"§via the PC's 8237A-5 DMA controller.  These methods are only available when the system is initialized for Aria PCM Audio and Aria Synthesizer.

### 2.4.1	Data Transfer Procedure

All digital audio data played or recorded by the audio system is transferred in data packets.  A data packet is a group of PCM samples corresponding to a specific digital audio channel.  The default packet size is the maximum of 512 words -- this could contain 512, 16-bit samples or 1024, 8-bit samples.

A transfer is initiated by the PC when it sends Command ID 11h (START AUDIO PLAYBACK) or Command ID 15h (START AUDIO RECORDING) to the DSP.  These commands are detailed in Section 3.5.2, Digital Audio Recording and Playback Commands.

Once a digital audio start command has been issued, interrupts will be generated by the DSP whenever data transfer is necessary.  The PC should then read the DSP DATA PORT to determine the interrupt source -- a value of 0001h (bit 0 set) indicates a PCM interrupt.  Data packets must be sent or received when this type of interrupt occurs on the PC.  One complete packet for each active channel must then be transferred during this interrupt time.  The data packet for each audio channel is transferred in an alternating fashion with two FIFO buffersXE "FIFO Buffers"§ resident in DSP RAM.

When playing digital audio, Command ID 10h (DIGITAL AUDIO TRANSFER) must be issued after data packets for all active channels have been sent.  The hardware interrupt need not be 'cleared' by the PC.

When playback or recording is completed, Command ID 12h (STOP AUDIO PLAYBACK) or Command ID 16h (STOP AUDIO RECORDING) must be sent.  Once all digital audio channels have been terminated, the DSP will cease sending interrupts to the PC.

XE "Data Transfer Procedure" \r "DataTransfer"§

### 2.4.2	Direct Input and Output

The fastest method of transferring digital audio data packets involves directly reading and writing DSP memory.  The data for each packet is transferred word-by-word using PC port reads or writes.  This method is preferred for all 80286 and above PCs, since you could make use of the INS and OUTS assembly language string instructions.

One data packet for each active audio channel can be transferred within the Interrupt Service Routine.  The drawing below shows an example stream of data for two channels:

µ §

The process for directly sending audio data to the DSP for playback is as followsXE "Digital Audio Playback"§:

1.	Be sure the system is set to the appropriate format by sending Command ID 03h (DIGITAL AUDIO FORMAT) and adjusting the DSP CONTROL Port to the required sample rate, if necessary.

2.	Send Command ID 11h (START AUDIO PLAYBACK) for each channel of digital audio to be output.

3.	When an interrupt occurs, the interrupt is from the DSP.  Check the interrupt type by reading the DSP DATA PORT -- Bit zero should be set, indicating PCM audio.  If not, exit the interrupt routine.  Data for each packet should now be written to one of the two DSP FIFO buffers designated for each audio channel.  After the packet for the last active channel is written, Command ID 10h (DIGITAL AUDIO TRANSFER) must be issued.

4.	To halt playback, send Command ID 12h (STOP AUDIO PLAYBACK) for each channel of digital audio.

The process for receiving audio data from the DSP while recording is as followsXE "Digital Audio Recording"§:

1.	Be sure the system is set to the appropriate format by sending  Command ID 03h (DIGITAL AUDIO FORMAT) and adjusting the DSP CONTROL Port to the required sample rate, if necessary.

2.	Send Command ID 15h (START AUDIO RECORDING).

3.	When an interrupt occurs, the interrupt is from the DSP.  Check the interrupt type by reading the DSP DATA PORT -- Bit zero should be set, indicating PCM audio.  If not, exit the interrupt routine.  Otherwise, the appropriate data packet should be read from the DSP FIFO buffer by the PC.

4.	To halt recording, send Command ID 16h (STOP AUDIO RECORDING).

Interrupt Service RoutineXE "Interrupt Service Routine"§

Data packet transfers are made from within an Interrupt Service Routine (ISR) on the PC.  The ISR must be installed to service hardware interrupt IRQ 10, 11 or 12 (software interrupts 72h, 73h or 74h respectively).  A coding example is provided on the developer disk demonstrating an ISR (in PCM_ISR.ASM).

XE "FIFO Buffers"§The first step is to specify to the DSP a source or destination address for the packet.  This is accomplished by writing a word to the DMA ADDRESS Port of the DSP.  The following addresses should be used if the packet size is 512:

| Packet set 1, 3, 5, . . . | Packet set 2, 4, 6, . . . |  |
| --- | --- | --- |
| 7C00h 7800h 7400h 7000h | 7E00h 7A00h 7600h 7200h | for audio channel 0 for audio channel 1 for audio channel 2 for audio channel 3 |

These addresses can also be derived by reading DSP RAM location 6100h for playback and 6101h for recording.  The value found at these locations will indicate an offset into the DSP audio data buffer.  The offset value toggles between 0 and the packet size, which allows the following equation to be implemented:

destination addr = 8000h - ((channel+1)*(packetsize*2))+offset

where offset is the value read from DSP RAM location 6100h or 6101h.

The drawing below shows an example of a data packet transfer with the appropriate destination addresses (a packet size of 512 is assumed).  Channel 0 is playing initially; channel 1 is being added.

µ §

NOTE:	  When recording, use only the address locations for audio channel 0.

After setting the source or destination address, each data word of the packet can be transferred as shown in the C language programming example below:

void transfer (address, *data, fn)

unsigned short address;	// DSP address

unsigned short *data;	// Buffer pointer

short fn;	// Function: 0=play, 1=record

{

short i;

_disable ();		// Disable PC interrupts

outpw (baseport+4, address);	// Set DSP address

// NOTE: Address will auto-increment

if (fn)		// Receive data
		for (i = 0; i < 512; ++i)

*(data+i) = inpw (baseport+6);

else			// or send data

for (i = 0; i < 512; ++i)

outpw (baseport+6, *(data+i));

_enable ();		// Enable interrupts

}

After the packet for the final active channel is written, Command ID 10h (DIGITAL AUDIO TRANSFER) must be issued to the DSP.

When exiting the Interrupt Service Routine, be sure to send an end-of-interrupt (EOI) to each PC interrupt controller:

outp (0xA0, 0x20);	// secondary controller EOI

outp (0x20, 0x20);	// primary controller EOI

XE "Direct Input and Output" \r "DirectIO"§

### 2.4.3	Direct Memory Access

Another method of transferring digital audio data packets is Direct Memory Access (DMA) using the 8237A-5 DMA Controller.  This technique is exactly the same as the one described in Section 3.4.2, Direct Input and Output, except only one 512 word data packet can be transferred with each DMA operation.  The drawing below shows an example stream of audio data for two channels:

µ §

The process for sending audio data to the DSP for playback is as followsXE "Digital Audio Playback"§:

1.	Be sure the system is set to the appropriate format by sending Command ID 03h (DIGITAL AUDIO FORMAT) and adjusting the DSP CONTROL Port to the required sample rate, if necessary.

2.	Send Command ID 11h (START AUDIO PLAYBACK) for each channel of digital audio to be output.

3.	When an interrupt occurs,

a)	First check the interrupt type by reading the DSP DATA PORT.  Bit zero should be set, indicating PCM audio.  If not, exit the interrupt routine.

b)	If a data packet for the first channel has not been sent, the interrupt is from the DSP.  Data for packet 1 should be sent via DMA.  The DSP CONTROL Port will be set to allow interrupts on DMA terminal count (See Interrupt Service Routine for details).

c)	Otherwise, the interrupt is due to DMA terminal count of the previous packet.  If the packet for the last active channel has already been sent, Command ID 10h (DIGITAL AUDIO TRANSFER) must now be sent.

4.	To halt playback, send Command ID 12h (STOP AUDIO PLAYBACK) for each channel of digital audio.

The process for recording audio via DMA is identical to the method described in Section 3.4.2, Direct Input and OutputXE "Digital Audio Recording"§.  Instead of reading a data packet word-by-word, the data must be read using the PC's DMA controller.

Interrupt Service RoutineXE "Interrupt Service Routine"§

As with Direct I/O transfers, an ISR must be installed to service hardware interrupt IRQ 10, 11 or 12 (software interrupts 72h, 73h or 74h, respectively).  In addition, the PC's DMA controller must be properly programmed for the data transfer.

First, a few words about the PC AT's DMA Controller.  All IBM PC AT computers (and compatibles) contain two 8237A-5 DMA Controller chips.  One controller handles the 
8-bit DMA channels 0 to 3 and the other handles 16-bit DMA channels 5 to 7.  The Aria system can utilize DMAC channel 5 or 6 for data transfers.

The DMA controller requires that data buffer addresses must be an absolute 'page:base' address, instead of the usual 'segment:offset' address.  As we will see, it is fairly straightforward to perform this conversion.

A programming example in C language is shown below:

unsigned char far *bufferptr;	// The data buffer pointer

unsigned long absaddr;	// The absolute address
unsigned short page, base, baseLSB, baseMSB;

addr = ((unsigned long) FP_SEG(bufferptr << 4) +
        (unsigned long) FP_OFF(bufferptr));
page = (unsigned short) (absaddr >> 16);
base = (unsigned short) ((absaddr >> 1) & 0x0000FFFF);
baseLSB = base & 0xFF;
baseMSB = (base >> 8) & 0xFF;

Take the segment address of the PC audio buffer, multiply it by 16 and add the offset.  You will need to make sure that 32-bit values are used for these operations.  The absolute page number will be the most significant word of the result and the absolute byte offset will be the least significant word.  The page number should be saved in a temporary variable and then the entire absolute address divided by 2, in order to derive the base word address for 16-bit transfers.

The following C language example shows how to initialize the DMA controller for digital audio playback or recording (assuming DMA channel 5):

unsigned short count=512;	// Packet size

outp (0xD4,0x05); 	// Mask DMA channel 5
outp (0xD8,0x00); 	// Clear pointer flip-flop
if (recording)
   outp (0xD6,0x45);	// Set record mode
else	//   or
   outp (0xD6,0x49);	// Set playback mode
outp (0xC4,baseLSB);	// Set base addr LSB
outp (0xC4,baseMSB);	// Set base addr MSB
outp (0x8B,page);	// Set page number
outp (0xC6,(count-1)&0x00FF);	// Set count LSB
outp (0xC6,(count-1) >> 8);	// Set count MSB
outp (0xD4,0x01);	// Enable DMA channel 5

1.	Output 05h to port D4h.  This masks DMA channel 5.

2.	Output 00h to port D8h.  This clears the byte pointer flip-flop.

3.	Output 49h to port D6h.  This sets the transfer mode for playback.  For recording, output 45h to port D6h.

4.	Output the base address LSB to port C4h.

5.	Output the base address MSB to port C4h.

6.	Output the page number to 8Bh.

7.	Output the LSB of (packet size - 1) to port C6h.  This would be FFh, if the default of 512 words is used.

8.	Output the MSB of (packet size - 1) to port C6h.  This would be 01h, if the default of 512 words is used.

9.	Output 01h to port D4.  This re-enables DMA channel 5.

The next step is to specify to the DSP a source or destination address for the packet.  This is accomplished by writing a word to the DMA ADDRESS Port of the DSP.  This word address is derived exactly the same as the method described in Section 3.4.2.

The DSP CONTROL Port must now be initialized to start the DMA transfer.  Set or clear the "DMA direction" bit according to the type of operation desired.  Set the "Initiate DMA transfer" bit and the "Interrupt PC on DMA terminal count" bit.  If only playing one channel of digital audio or recording digital audio, the interrupt on DMA terminal count is not required -- as no action within the ISR is necessary upon receiving a DMA terminal count interrupt.

PLEASE NOTE:  Once a DMA transfer has been initiated, no additional values may be written to the DSP's DMA ADDRESS Port.  This means that DSP memory locations cannot be read or written until a DMA transfer is complete.

When exiting the ISR, be sure to send an end-of-interrupt (EOI) to each PC interrupt controller:

outp (0xA0, 0x20);	// secondary controller EOI

outp (0x20, 0x20);	// primary controller EOIXE "Digital Audio Data Transfer" \r "DigitalAudio"§

## XE "DMA" \r "DMA"§

## 2.50	DSP COMMANDS

The audio system responds to command sequences written to the DSP 16-BIT DATA Port (see 3.1.1, DSP Registers).  These commands control system configuration, digital audio recording and playback, and the Aria Synthesizer.

A command sequence consists of a series of 16-bit words that start with an unique command identifier and end with a standard command terminator (FFFF hex).

| 0000 | xxxx |  | . . . |  | yyyy | FFFF |
| --- | --- | --- | --- | --- | --- | --- |
| ID | data |  |  |  | data | term. |

Values printed inside of the command sequence boxes are in hexidecimal.  
Required variable values are denoted by x and y.

After each individual word of a command sequence is written to the DSP DATA Port, the PC must wait a minimum of 10 microseconds before writing another word.  A better method of detecting when the next word can be written is to read the DSP STATUS Port and check the value of bit 15 (Port 0 Busy flag).  If this bit is clear (0), the next word may be written.  If the bit is set (1), the next word may not be written and the status port should be polled again.

PC interrupts must be disabled while sending a command sequence.  This prevents conflict with any interrupt-driven DSP command routines.  The following is a C language example demonstrating how to send a command sequence:

// Function to send one command word (Returns 0 if ok, 1 if error)

int sendcmd (unsigned val)

{

unsigned i=65535;

// Time-out loop

while (i--)

{

// Check DSP Busy flag

if (!(inpw(baseport+2) & 0x8000))

{

outpw (baseport, val);	// Send value

return 0;

}

}

return 1;

}

// Code to send a DIGITAL AUDIO TRANSFER command string:

int status=0;

_disable ();		// Disable interrupts

status |= sendcmd (0x0003);	// Send Command ID

status |= sendcmd (0x0010);	// Send Data

status |= sendcmd (0xFFFF);	// Send Terminator

_enable ();		// Enable interrupts

// if status > 0, error has occurred

### 2.5.1	System Configuration Commands

This section describes all commands used to initialize and configure the audio system.  Included are the following commands:

| Command ID 00h 02h 03h 04h 05h 06h 07h 08h 09h 0Ah | Command Name System Initialization Download Wave Parameter Table Digital Audio Format Master Volume Digital Audio Packet Size Playback Mode Auxiliary Volume External Volume Mixer Configuration Generate PC Interrupt |
| --- | --- |

XE "COMMANDS:System Initialization"§System InitializationXE "System Initialization"§	Command ID 00h

This command initializes and invokes DSP applications and modules.

| 0000 | 00xx | xxyy | xxxx | FFFF |
| --- | --- | --- | --- | --- |
| ID | cmd | task # | addr |  |

The command (cmd) value indicates the operation to perform:

00h	Add task to DSP execution list

01h	Remove task from DSP execution list

02h	Load new module for DSP task #0

The task number is a DSP application or module number:

xx	00h = ROM-based task or module

01h = RAM-based task or module

The currently defined ROM-based tasks are:

yy	00h = Aria PCM Audio and Aria SynthesizerXE "Aria Synthesizer"§

01h = Sound Blaster Emulation (default)XE "Sound Blaster Emulation"§
	05h = Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§
	07h = ReverbXE "Reverb"§ Effects

The address parameter (addr) is the entry point address for RAM-based DSP applications and modules.  If not required, a zero value should be sent.

Most PC applications will only need to switch between ROM-based DSP applications.  Since the system default is Sound Blaster Emulation, a SYSTEM INITIALIZATION command must be sent for "Aria PCM Audio and Aria Synthesizer," if used.

Example (initialization for Aria mode):

0000h	0000h	0000h	0000h	FFFFh

The system should be returned to Sound Blaster mode when the application is finished.

Example (initialization for Sound Blaster mode):

0000h	0000h	0001h	0000h	FFFFh

IMPORTANT:

Bit 7 of the DSP CONTROL Port must be set to Aria Synthesizer mode before switching any applications.  After sending an initialization command sequence, the DSP CONTROL Port must then be set to reflect the desired synthesis method.

Download Wave Parameter Table	Command ID 02h

This command is used to download a Wave Parameter Table.  A Wave Parameter Table defines an amplitude envelope, LFO parameters and an initial stereo pan value for the Aria Synthesizer.  When a synthesis operator is turned on using an OPERATOR ON command (Command ID 20h or Command ID 23h), it must reference a Wave Parameter Table.

| 0002 | 00xx | xxxx | xxxx | xxxx | xxxx | xxxx | xxxx | xxxx | xxxx | . . . |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | WPT # | env1 | env2 | env3 | env4 | env5 | env6 | env7 | env8 |  |

| . . . | 000x | 00xx | 000x | 00xx | 000x | 00xx | 00xx | FFFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LFO # | depth | LFO # | depth | LFO # | depth | pan |  |
|  | ¦__________¦ Pitch LFO |  | ¦__________¦ Amplitude LFO |  | ¦___________¦ Pan LFO |  |  |  |

WPT #	Wave Parameter Table number (00h to 4Fh)

env1-env8	Eight-stage amplitude envelope definition.  When an operator is turned on, stages env1 to env8 will execute.  When the operator is turned off, a jump to stage env7 will occur.  Each envelope stage parameter is defined as follows:

Bits 0 to 5:	Envelope Index (00h to 3Fh)

Bits 6 to 15:	Envelope Length (000h to 3FEh)

(see additional notes on next page)

LFO #	Low Frequency Oscillator numberXE "Low Frequency Oscillator"§ (00h to 0Fh).

depth	Amount of LFO effect (00h is off, FFh is maximum).

pan	Starting pan value (00h is full left, 40h is center, and 7Fh is full right).

The Envelope IndexXE "Envelope"§ defines how rapidly the operator amplitude will increase or decrease over time.  The following table shows all indices and their corresponding rate of change:

| Attack Index | -96 dB to 0 dB in seconds |  |  | Decay Index | 0 dB to -96dB in seconds |
| --- | --- | --- | --- | --- | --- |
| 0 | Sustain |  |  |  |  |
| 1 | 5.6691 |  |  | 32 | 83.8984 |
| 2 | 3.7910 |  |  | 33 | 54.2816 |
| 3 | 2.8447 |  |  | 34 | 46.1393 |
| 4 | 2.2642 |  |  | 35 | 30.7547 |
| 5 | 1.6081 |  |  | 36 | 25.6255 |
| 6 | 1.4224 |  |  | 37 | 19.2163 |
| 7 | 0.9492 |  |  | 38 | 10.7199 |
| 8 | 0.7141 |  |  | 39 | 9.4049 |
| 9 | 0.5689 |  |  | 40 | 7.4920 |
| 10 | 0.4761 |  |  | 41 | 5.3237 |
| 11 | 0.4035 |  |  | 42 | 3.7242 |
| 12 | 0.3570 |  |  | 43 | 3.0943 |
| 13 | 0.2380 |  |  | 44 | 2.3193 |
| 14 | 0.2032 |  |  | 45 | 1.3237 |
| 15 | 0.1800 |  |  | 46 | 0.9260 |
| 16 | 0.1190 |  |  | 47 | 0.7721 |
| 17 | 0.1016 |  |  | 48 | 0.6618 |
| 18 | 0.0900 |  |  | 49 | 0.3309 |
| 19 | 0.0726 |  |  | 50 | 0.2903 |
| 20 | 0.0522 |  |  | 51 | 0.2322 |
| 21 | 0.0464 |  |  | 52 | 0.1655 |
| 22 | 0.0377 |  |  | 53 | 0.1161 |
| 23 | 0.0319 |  |  | 54 | 0.0842 |
| 24 | 0.0261 |  |  | 55 | 0.0493 |
| 25 | 0.0232 |  |  | 56 | 0.0377 |
| 26 | 0.0203 |  |  | 57 | 0.0232 |
| 27 | 0.0174 |  |  | 58 | 0.0203 |
| 28 | 0.0145 |  |  | 59 | 0.0145 |
| 29 | 0.0116 |  |  | 60 | 0.0116 |
| 30 | 0.0100 |  |  | 61 | 0.0100 |
| 31 | 0.0000 |  |  | 62 | 0.0058 |

Table 3.1  ENVELOPE INDEX RATES

An Envelope Index of 31 specified for the first envelope stage, will cause an operator to turn on at full amplitude when an OPERATOR ON (Command ID 20h or Command ID 23h) is sent.  This is used for sampled percussion waveforms and sampled attack waveforms.

The Envelope Length XE "Envelope"§defines the amount of time spent in one envelope stage before proceeding to the next stage.  Each length unit is 2.9 milliseconds in duration.  The maximum length is 1022 units (up to 3 seconds).

If the Envelope Length is set to zero, this indicates an immediate GOTO or jump to another envelope stage.  In this case, the destination stage (0 to 7) must be specified by the Envelope Index.  A GOTO is used for looping a sustain phase of the envelope or extending the duration of an attack or decay.

Example

The following data defines a typical amplitude envelope that has a moderate attack and decay (stages 0 and 1), a looping sustain (stages 2 and 3), and a moderate release (stage 6).

| STAGE | INDEX | LENGTH |
| --- | --- | --- |
| 0 | 10 | 95 |
| 1 | 52 | 60 |
| 2 | 0 | 10 |
| 3 | 2 | 0 |
| 4 | 0 | 1 |
| 5 | 0 | 1 |
| 6 | 48 | 145 |
| 7 | 0 | 1 |

µ §

Note that stage 3 is used as a GOTO to create a sustaining sound.  Upon receiving the OPERATOR OFF command, stage 6 will be invoked.  Stages 4 and 5 are essentially unused in this example.  Stage 7 also has no audible effect.

XE "COMMANDS:Download Wave Parameter Table" \r "WPT"§XE "Wave Parameter Table" \r "WPT"§

Digital Audio FormatXE "COMMANDS:Digital Audio Format"§	Command ID 03h

This command sets the data format for playing and recording digital audio.

| 0003 | 00xy | FFFF |
| --- | --- | --- |
| ID | format |  |

The data format can be a combination of the following values:

| Sample rateXE "Sample Rate"§ (x) |  | Data format (y) |
| --- | --- | --- |
| 0yh 11.025 KHz |  | x0h 8-bit mono (default) |
| 1yh 22.050 KHz |  | x1h 8-bit stereo |
| 2yh 44.100 KHz* |  | x2h 16-bit mono |
|  |  | x3h 16-bit stereo |
|  |  | x4h ADPCMXE "ADPCM Compression"§ compressed (4:1), mono |
|  |  | x5h ADPCM compressed (4:1), stereo |
| NOTE: THESE | Þ | x6h m-Law companded, mono |
| DATA FORMATS |  | x7h m-Law companded, stereo |
| NOT AVAILABLE |  | x8h A-Law companded, mono |
| WITH SC18025 DSP |  | x9h A-Law companded, stereo |

*	When selecting a sample rate of 44.1 kHz, the PLAYBACK MODE (Command ID 06h) must be set to 0 and the DSP CONTROL Port must be set for 44.1 KHz sample rate.

For example, if format 0001h is specified, the audio device will be set to playback audio at 11.025 kHz and the data will be processed as 8-bit stereo (interleaved -- left channel, right channel).

All 8-bit samples must be unsigned data and all 16-bit samples must be signed data (two's complement), least significant byte first.

When using Adaptive Delta Pulse Code Modulation (ADPCM) XE "ADPCM Compression"§during recording or playback, the system PLAYBACK MODE must be set to 0 (No Aria Synthesizer), the sample rate must be set to 22 KHz, and only digital audio channel 0 may be used.  Simultaneous ADPCM recording and playback is not supported.

When recording, the ADPCM algorithm will compress 16-bit samples into 4-bit samples.  ADPCM decompression during playback will accept the 4-bit samples and reproduce a 16-bit output.  Four, 4-bit samples are packed into each 16-bit word.

The m-LawXE "Mu-Law Companding"§ and A-LawXE "A-Law Companding"§ companded data modes utilize 8-bit data samples, but yield greater dynamic range than ordinary 8-bit PCM samples.  Please note that these modes are not available with the SC18025 DSP.

Master VolumeXE "COMMANDS:Master Volume"§XE "Master Volume"§XE "Volume:Master"§	Command ID 04h

This command sets the left and right channel master volume for the entire audio device.*

| 0004 | xxxx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

The left and right volume levels are linear and can be specified from 0000h (off) to 7FFFh (max. and default).

*For Aria systems with the SC18075 Mixer chip, the master volume command will control the entire audio output.  For other systems, the command will only effect the Aria Synthesizer and Aria PCM audio outputs.  Also when recording with these other systems, the master volume command will attenuate the recording level if the input monitor has been enabled.  See Command ID 09h, MIXER CONFIGURATION for more information on input monitor selection.

Digital Audio Packet SizeXE "COMMANDS:Digital Audio Packet Size"§	Command ID 05h

This command sets the number of 16-bit words that comprise a digital audio data packet to be sent or receivedXE "FIFO Buffers"§.  Any active digital audio channels will be turned off, when this command is sent.

| 0005 | 0xxx | FFFF |
| --- | --- | --- |
| ID | size |  |

Only the following size values are valid:  4, 8, 16, 32, 64, 128, 256, and 512.  The default packet size is 512 words.

When 8-bit values are sent or received by the PC, each packet must contain twice as many data samples as specified by the packet size.  This is due to the fact that two 8-bit byte values will be packed into each 16-bit word that is transferred.

Playback ModeXE "COMMANDS:Playback Mode"§	Command ID 06h

This command initializes the audio system into one of three playback modes.  PCM audio channels and Aria Synthesizer operators should be inactive when this command is issued.

| 0006 | 000x | FFFF |
| --- | --- | --- |
| ID | mode |  |

The mode value can be one of the following:

0	4 mono digital audio channels (Channels 0, 1, 2 and 3) or
2 stereo digital audio channels (Channels 0 and 1)
	- with -
No synthesis operators

1	2 mono digital audio channels (Channels 0 and 1) or
1 stereo digital audio channel (Channel 0)
	- with -
20 Aria Wave Synthesis operators (Operators 0 to 19)

2	Default
2 mono digital audio channels (Channels 0 and 1) or
1 stereo digital audio channel (Channel 0)
	- with -
32 Aria Wave Synthesis operators (Operators 0 to 31)

The digital audio channels listed above may be used for either recording or playback.

Mode 1 is a low-end Aria Synthesizer reserved for simultaneous use with Aria ListenerXE "Aria Listener"§XE "Speech Recognition"§, reverbXE "Reverb"§ processing or QSound Virtual AudioXE "QSound Virtual Audio"§ effects..

CD-ROM VolumeXE "COMMANDS:Auxiliary Volume"§XE "Auxiliary Volume"§XE "Volume:Auxiliary"§XE "COMMANDS:CD-ROM Volume"§XE "CD-ROM Volume"§XE "Volume:CD-ROM"§	Command ID 07h

This command sets the left and right channel auxiliary input volume for the Aria system.  The auxiliary input is usually connected to a CD-ROM audio output.  This command will also effect the recording level when recording from the CD-ROM audio input.

| 0007 | xxxx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

The left and right volume levels are linear and can be specified from 0000h (off) to 7FFFh (max).  Default is 6400h.

When recording monophonic signals, the left volume level will be used as the overall recording level.  The right volume level must always be specified, even when not in use.

To listen to the CD-ROM signal mixed with the output, the CD-ROM monitor must also be enabled (see Command ID 09h, MIXER CONFIGURATION).

Microphone VolumeXE "COMMANDS:External Volume"§XE "External Volume"§XE "Volume:External"§XE "COMMANDS:Microphone Volume"§XE "Microphone Volume"§XE "Volume:Microphone"§	Command ID 08h

This command sets the left and right channel external input volume for the Aria system.  The external input is usually connected to a microphone or line in audio source.  This command will also effect the recording level when recording from the external input.

| 0008 | xxxx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

The left and right volume levels can be specified from 0000h (off) to 7FFFh (max).  Default is 6400h.

When recording monophonic signals, the left volume level will be used as the overall recording level.  The right volume level must always be specified, even when not in use.

Aria systems with the SC18075 Mixer chip contain seperate microphone and line level external inputs.  In this case, the command will effect only the microphone input.  Other systems can use Command ID 09h, MIXER CONFIGURATION, to select between microphone and line level input.

To listen to this external signal mixed with the output, the microphone monitor must also be enabled (see Command ID 09h, MIXER CONFIGURATION).

Mixer ConfigurationXE "Mixer"§XE "COMMANDS:Mixer Configuration"§XE "Mixer Commands"§	Command ID 09h

This command selects the recording sourceXE "Recording Source"§, input level and monitor settingsXE "Monitor"§ for the audio system mixer.  Input from the external source can be switched between microphone and line level.  Signals from the microphone input and auxiliary input (CD-ROM audio) can be monitored with the output in mono or stereo.

| 0009 | 000x | 0xyz | 0xy0 | FFFF |
| --- | --- | --- | --- | --- |
| ID | ADC | mic. in | aux. in |  |

ADC	ADCXE "ADC"§ recording source select (x, bits 0-2):
	1 = Microphone source (default)
	2 = Auxiliary source (CD-ROM audio)
	4 = Aria audio system output (synth + PCM audio)

mic. in	Microphone monitor mode (x, bit 8):
	0 = Stereo (default)
	1 = Mono

Microphone monitor enable (y, bit 4):
	0 = Disabled (default)
	1 = Enabled (external input signal will be audible)

Microphone input select (z, bit 0):
	0 = Line level
	1 = Microphone level (default)

aux. in	Auxiliary CD-ROM monitor mode (x, bit 8):
	0 = Stereo (default)
	1 = Mono

Auxiliary CD-ROM monitor enable (y, bit 4):
	0 = Disabled
	1 = Enabled (auxiliary input signal will be audible, default)

Aria systems with the SC18075 Mixer chip may also use Command IDs 30h, 32h, and 33h for controlling the additional mixer configurations that are possible.

Generate PC InterruptXE "COMMANDS:Generate PC Interrupt"§XE "Interrupts"§XE "IRQ"§	Command ID 0Ah

This command causes a hardware interrupt (IRQ 10, 11 or 12) to be generated on the PC.

| 000A | FFFF |
| --- | --- |
| ID |  |

If the Aria board is jumpered for IRQ 10, 11 or 12, software interrupt 72h, 73h or 74h will be generated, respectively.

By temporarily installing a short interrupt service routine at each possible software interrupt vector, this command can be used to 'auto-detect' how the hardware jumper has been set.

XE "System Configuration Commands" \r "ConfigCommands"§

### 2.5.2	Digital Audio Recording and Playback Commands

This section describes all commands used to control digital audio recording and playback.  Up to four separate channels of digitized audio may be output and one stereo channel may be input.

The following commands are included in this section:

| Command ID 10h 11h 12h 13h 14h 15h 16h 17h | Command Name Digital Audio Transfer Start Audio Playback Stop Audio Playback Playback Channel Volume Playback Channel Pan Start Audio Recording Stop Audio Recording Digital Audio Playback Volume |
| --- | --- |

Digital Audio TransferXE "COMMANDS:Digital Audio Transfer"§	Command ID 10h

This command signals the completion of transferring a set of digital audio packets to the audio system for playback.  The command is issued after packets for all active channels have been sent.

| 0010 | FFFF |
| --- | --- |
| ID |  |

For more information, see Section 3.4 DIGITAL AUDIO DATA TRANSFER.

Start Audio PlaybackXE "COMMANDS:Start Audio Playback"§XE "Playback"§	Command ID 11h

This command activates an audio playback channel.  This command must be sent prior to sending the digital audio packets for that channel.

| 0011 | 000x | FFFF |
| --- | --- | --- |
| ID | chan. |  |

The channel number can be 0, 1, 2 or 3.

When playing a stereo file, only channel 0 or channel 1 may be used.

The number of available playback channels is dependent on the current playback mode.  For more information, refer to the PLAYBACK MODE command (Command ID 06h).

Stop Audio PlaybackXE "COMMANDS:Stop Audio Playback"§	Command ID 12h

This command deactivates an audio playback channel.  Digital audio packets corresponding to this channel should not be sent after issuing this command.

| 0012 | 000x | FFFF |
| --- | --- | --- |
| ID | chan. |  |

The channel number must be 0, 1, 2 or 3.

Playback Channel VolumeXE "COMMANDS:Playback Channel Volume"§XE "Volume:Playback Channel"§	Command ID 13h

This command sets the maximum amplitude or volume of a specific audio playback channel.

| 0013 | 000x | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | chan. | volume |  |

The channel number must be 0, 1, 2 or 3.

Volume is linear and can range from 0000h (off) to 7FFFh (max. and default).

For adjusting the overall playback volume, see DIGITAL AUDIO PLAYBACK VOLUME (Command ID 17h).

Final output master volume is controlled with the MASTER VOLUME command (Command ID 04h).

Playback Channel PanXE "COMMANDS:Playback Channel Pan"§	Command ID 14h

This command sets the stereo pan of a specific audio playback channel.

| 0014 | 000x | 00xx | FFFF |
| --- | --- | --- | --- |
| ID | chan. | pan |  |

The channel number must be 0, 1, 2 or 3.

The pan value can range from 00h (full left) to 40h (center and default) to 7Fh (full right).

Start Audio RecordingXE "COMMANDS:Start Audio Recording"§XE "Recording"§	Command ID 15h

This command starts recording digital audio data and will initiate interrupts on the PC when data is ready to be received.  Only one recording channel (mono or stereo) may be active at any given time.

| 0015 | 000x |  | FFFF |
| --- | --- | --- | --- |
| ID | chan. |  |  |

The channel number must be 0 or 1.  Any active playback channels must use a different channel number.

The recording source is selected using the MIXER CONFIGURATION command (Command ID 09h).

For information on recording digital audio, see Section 3.4, DIGITAL AUDIO DATA TRANSFER.

Stop Audio RecordingXE "COMMANDS:Stop Audio Recording"§	Command ID 16h

This command halts all recording of digital audio data.  No further data packet interrupt requests will be generated on the PC.

| 0016 | 000x | FFFF |
| --- | --- | --- |
| ID | chan. |  |

The channel number must be 0 or 1.

Digital Audio Playback VolumeXE "COMMANDS:Digital Audio Playback Volume"§XE "Volume:Digital Audio Playback"§	Command ID 17h

This command sets the left and right output volumes for all digital audio playback channels.

| 0017 | xxxx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

The left and right volume levels can be specified from 0000h (off) to 7FFFh (max. and default).

For adjusting individual playback channel volumes, see PLAYBACK CHANNEL VOLUME (Command ID 13h).

Final output master volume is controlled with the MASTER VOLUME command (Command ID 04h).XE "Digital Audio Recording and Playback Commands" \r "RecAndPlay"§

### 2.5.3	Aria Synthesizer Commands

This section describes all commands used to control the Aria Synthesizer operators.  Included are:

| Command ID 20h 21h 22h 23h 24h 25h 26h 27h 28h 29h 2Ah 2Bh 2Ch | Command Name Operator On (Loop) Operator Off Operator Cancel Operator On (Single Play) LFO Activate LFO Deactivate Amplitude Change Pan Change Pitch Change Synthesis Volume LFO Rate Change LFO Depth Change Table Modify |
| --- | --- |

Operator On (Loop)XE "COMMANDS:Operator On (Loop)"§	Command ID 20h

This command activates a synthesis operator and initiates the attack phase of the specified amplitude envelope.  The waveform will loop (play continuously) until an Operator Off or Operator Cancel command is sent.

| 0020 | 00xx | 00xx | 0xxx | xxxx | xxyy | 0xxx | FFFF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID | op # | WPT # | wave # | amp | pitch | size |  |

op #	Operator number (00h to 1Fh).

WPT #	Wave Parameter Table number (00h to 4Fh).

wave #	Wave Block number (0000h to FFFFh).

amp	Operator maximum amplitude (0000h to 7FFFh).

pitch	xx	Operator pitch - integer portion (00h to FEh).
yy	Operator pitch - fractional portion (00h to FFh).

(see additional notes below)

size	Number of 256 byte Wave Blocks to output (001h to 100h).

Operator pitch is computed using the following equations:

xx = (int)

Note:	Freq (in Hz) is the desired output frequency.
Base (in Hz) is the frequency of the Wave Block when output at 22.05 KHz:

Base = 	where, Size is the number of 256 byte Wave Blocks

All of the necessary parameters for determining the operator frequency are stored in Aria patch bank files.

Operator On, Example 1: 

To produce a frequency of 440.00 Hz using a Wave Block of size 2 (512 samples), 
Base Frequency is 22050 / 512 = 43.066 Hz and would yield a value of 0Ah for xx and 37h for yy.

xx = (int)  = 10 (0Ah)

 = 55 (37h)

Operator On, Example 2:

To produce a frequency of 440.00 Hz using a Wave Block of size 4 (1024 samples), 
Base Frequency is 22050 / 1024 = 21.533 Hz and would yield a value of 14h for xx and 6Fh for yy.

xx = (int)  = 20 (14h)

 = 111 (6Fh)

XE "COMMANDS:Operator On" \r "OperatorOn"§XE "Operator" \r "OperatorOn"§

Operator OffXE "COMMANDS:Operator Off"§	Command ID 21h

This command turns off the specified operator by causing the amplitude envelope to enter the release phase (stages 7 and 8).

| 0021 | 00xx | FFFF |
| --- | --- | --- |
| ID | op # |  |

The operator number can range from 00h to 1Fh.

Operator CancelXE "COMMANDS:Operator Cancel"§	Command ID 22h

This command immediately turns off the specified operator. The amplitude envelope release phase will be ignored.

| 0022 | 00xx | FFFF |
| --- | --- | --- |
| ID | op # |  |

The operator number can range from 00h to 1Fh.

Operator On (Single Play)XE "COMMANDS:Operator On"§	Command ID 23h

This command activates a synthesis operator and initiates the attack phase of the specified amplitude envelope.  The waveform will not loop continuously, but play through only once.

| 0023 | 00xx | 00xx | 0xxx | xxxx | xxyy | xxxx | FFFF |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ID | op# | WPT# | wave# | amp | pitch | env # |  |

op #	Operator number (00h to 1Fh).

WPT #	Wave Parameter Table number (00h to 4Fh).

wave #	Wave Block number (0000h to FFFFh).

amp	Operator maximum amplitude (0000h to 7FFFh).

pitch	xx	Operator pitch - integer portion (00h to FEh).
yy	Operator pitch - fractional portion (00h to FFh).

Operator pitch is computed with the same equations as listed by the OPERATOR ON Loop command (see Command ID 20h).

env #	Number of amplitude envelope update periods before operator should turn off (0001h to FFFEh).  Use the following calculation:

			(int)  

size is the number of 256 byte Wave Blocks to output (001h to 100h).

XE "Operator" \r "Operator1"§

LFO ActivateXE "COMMANDS:LFO Activate"§	Command ID 24h

This command turns on a specified Low Frequency Oscillator.  The wave parameter table is used to link LFO's to specific operators.

| 0024 | 000x | 000x | xxyy | FFFF |
| --- | --- | --- | --- | --- |
| ID | LFO # | type | rate |  |

LFO #	LFO number to activate, 00h to 0Fh

type	0	Ramp
1	Reverse Ramp
2	Triangle
3	Square
4	Pulse (25% duty cycle)
5	Staircase (8 steps)
6	Sine Wave

rate	xx	LFO rate (integer portion), 00h to 3F
yy	LFO rate (fractional portion), 00h to FFh

LFO rate is computed using the following equations:

xx = (int)

Note:  Frequency is in Hertz.

For example, a frequency of 20.0 Hz would yield a value of 00h for xx and 3Bh for yy.

LFO DeactivateXE "COMMANDS:LFO Deactivate"§	Command ID 25h

This command turns off the specified Low Frequency Oscillator.

| 0025 | 000x | FFFF |
| --- | --- | --- |
| ID | LFO # |  |

The LFO number can range from 00h to 0Fh.XE "Low Frequency Oscillator" \r "LFO1"§

Amplitude ChangeXE "COMMANDS:Amplitude Change"§XE "Volume:Synthesis Operator"§	Command ID 26h

This command changes the maximum amplitude or volume of an active synthesis operator.

| 0026 | 00xx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | op | volume |  |

The operator number can range from 00h to 1Fh.  Volume is linear and can range from 0000h (off) to 7FFFh (max. and default).

For adjusting overall synthesis volume of all operators, see SYNTHESIS VOLUME (Command ID 29h).

Final output master volume is controlled with the MASTER VOLUME command (Command ID 04h).

Pan ChangeXE "COMMANDS:Pan Change"§	Command ID 27h

This command changes the stereo pan of an active synthesis operator.

| 0027 | 00xx | 00xx | FFFF |
| --- | --- | --- | --- |
| ID | op | pan |  |

The operator number can range from 00h to 1Fh.  Pan can range from 00h (full left) to 40h (center and default) to 7Fh (full right).

Pitch ChangeXE "COMMANDS:Pitch Change"§	Command ID 28h

This command changes the pitch of an active synthesis operator allowing for a "pitch bend" effect without restarting the operator.

| 0028 | 00xx | xxyy | FFFF |
| --- | --- | --- | --- |
| ID | op | pitch |  |

The operator number can range from 00h to 1Fh.  The pitch value is determined identical to the method used by the OPERATOR ON Loop command (Command ID 20h).

Synthesis VolumeXE "COMMANDS:Synthesis Volume"§XE "Volume:Synthesis"§	Command ID 29h

This command sets the left and right output volume for all synthesis operators.

| 0029 | xxxx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

The left and right volume levels are linear and can be specified from 0000h (off) to 7FFFh (max. and default).

For adjusting individual operator volume, see AMPLITUDE CHANGE (Command ID 26h).

Final output master volume is controlled with the MASTER VOLUME command (Command ID 04h).

LFO Rate ChangeXE "COMMANDS:LFO Rate Change"§	Command ID 2Ah

This command sets the rate of a specific LFO generator allowing for a modulation change without restarting the LFO.

| 002A | 000x | xxyy | FFFF |
| --- | --- | --- | --- |
| ID | LFO # | rate |  |

The LFO number can range from 00h to 0Fh.  The rate value determines the LFO rate identical to the method used by the LFO ACTIVATE command (Command ID 24h).

For adjusting an LFO depth, see LFO DEPTH CHANGE (Command ID 2Bh).

LFO Depth ChangeXE "COMMANDS:LFO Depth Change"§	Command ID 2Bh

This command sets the depth of a specific LFO generator allowing for a modulation change without retransmitting a wave parameter table.

| 002B | 00xx | 000x | 00xx | FFFF |
| --- | --- | --- | --- | --- |
| ID | WPT # | type | depth |  |

WPT #	Wave Parameter Table number (00h to 4Fh)

type	0	Pitch LFO (vibrato)
1	Amplitude LFO (tremolo)
2	Pan LFO

depth	Amount of LFO effect (00h is off, FFh is maximum)

For adjusting an LFO rate, see LFO RATE CHANGE (Command ID 2Ah).XE "Low Frequency Oscillator" \r "LFO2"§

Table ModifyXE "COMMANDS:Table Modify"§XE "Table Modify"§	Command ID 2Ch

This command directly modifies the selected audio system data table.

| 002C | 000x | 00xx | 00xx | xxxx | FFFF |
| --- | --- | --- | --- | --- | --- |
| ID | type | table # | offset | value |  |

type	0	Wave Parameter Tables
	Contains parameters defining the amplitude envelope.

1	Envelope generation multiplier table
	Contains multiplier values corresponding to amplitude envelope indices.

table #	Wave Parameter Tables (00h to 4Fh)
Envelope generator multiplier table (00h only)

offset	For Wave Parameter Tables:
00h-07h	Eight stage envelope parameters
0Eh		Right channel amplitude value (0 to 0x7FFF)
0Fh		Left channel amplitude value (0 to 0x7FFF)

Envelope generator multiplier table:
00h-3Fh	Multiplier values for envelope scaling

value	New data value to set  (NOTE:  value FFFFh is not allowed)

XE "Aria Synthesizer Commands" \r "SynthCommands"§

### 2.5.4	Mixer Commands

This section describes all commands used to control the SC18075 mixer chip.  
NOTE:  Systems without this mixer chip CANNOT ACCEPT these commands.

The following commands are included in this section:

| Command ID 30h 31h 32h 33h 34h 35h | Command Name Select ADC Source ADC Control Input Monitor Mode Master Monitor Mode Mixer Volume Levels Tone Control |
| --- | --- |

Select ADC SourceXE "COMMANDS:Select ADC Source"§	Command ID 30h

This command selects a recording source for the analog-to-digital convertors.

| 0030 | 00xx | FFFF |
| --- | --- | --- |
| ID | source |  |

The source parameter can be one of the following:

0000h	Squelch input

0001h	Microphone input
0002h	CD-ROM audio input

0004h	DAC input (Aria Synthesizer + PCM audio)

0008h	External (line level) input

0010h	Auxiliary input

0020h	PC Speaker/Telephone input

The ADC supports multiple sources simultaneously.  Only the following combinations are valid:

0005h	Microphone and DAC input

0003h	Microphone and CD-ROM audio

0021h	Microphone and PC Speaker/Telephone input

0014h	Auxiliary and DAC input

0012h	Auxiliary and CD-ROM audio

0030h	Auxiliary and PC Speaker/Telephone input

000Ch	External and DAC input

000Ah	External and CD-ROM audio

0028h	External and PC Speaker/Telephone input

WARNING:  Aria systems without the SC18075 Mixer chip cannot process this command.  DO NOT send this command unless you are sure of the Aria hardware configuration.

ADC ControlXE "COMMANDS:ADC Control"§	Command ID 31h

This command switches the Aria analog-to-digital convertor (ADC) on or off.

| 0031 | 000x | FFFF |
| --- | --- | --- |
| ID | mode |  |

The mode parameter should be set to 0 for ADC off or 1 for ADC on.xe "ADC" \r "ADC"§

WARNING:  Aria systems without the SC18075 Mixer chip cannot process this command.  DO NOT send this command unless you are sure of the Aria hardware configuration.

Input Monitor ModeXE "COMMANDS:Input Monitor Mode"§	Command ID 32h

This command can set each input channel monitor mode to mono or stereo.

| 0032 | 00xx | 000y | FFFF |
| --- | --- | --- | --- |
| ID | source | mode |  |

source	Value 	Source			
0x0001	Microphone input
0x0002	CD-ROM audio input
0x0004	Reserved
0x0008	External input (line level) 
0x0010	Auxiliary input (mixed output)
0x0020	PC Speaker/Telephone input

- plus -
0x8000	ORed with any of the above to select the recording monitor 	mode

mode	0	Monitor in stereo
1	Monitor in mono -- left channel
2	Monitor in mono -- right channel
3	Monitor off

NOTE:  The PC Speaker/Telephone input mode may only be set to mono -- left channel.

Example

A command string of:   0032h   0002h   0000h   FFFFh

would set the CD-ROM audio inputs to stereo mode.

WARNING:  Aria systems without the SC18075 Mixer chip cannot process this command.  DO NOT send this command unless you are sure of the Aria hardware configuration.

Master Monitor ModeXE "COMMANDS:Master Monitor Mode"§	Command ID 33h

This command sets the master output monitor mode (mono or stereo) and the recording mix connection.

| 0033 | 0x0y | FFFF |
| --- | --- | --- |
| ID | mode |  |

Bit 0 of the mode parameter must be set to 0 for stereo (default) or 1 for mono.  
Bit 8 of the mode parameter must be set to 0 for disconecting the recording mix output or 1 for connecting the recording mix output.  This is used to make recordings of the final mixed output via the Auxiliary input channel.

WARNING:  Aria systems without the SC18075 Mixer chip cannot process this command.  DO NOT send this command unless you are sure of the Aria hardware configuration.

Mixer Volume LevelsXE "COMMANDS:Mixer Volume Levels"§XE "Mixer Volumes"§	Command ID 34h

This command can set each input and output channel volume for the mixer.  All volume levels follow a logarithmic scale.

| 0034 | 00xx | xxxx | xxxx | FFFF |
| --- | --- | --- | --- | --- |
| ID | source | left | right |  |

source	Volume source must be one of the following values:
	0001h	Microphone input
	0002h	CD-ROM audio
	0004h	Master output
	0008h	External (line level) input
	0010h	Auxiliary input
	0020h	PC Speaker/Telephone input

left	Left channel volume (0000h to 7FFFh).

right	Right channel volume (0000h to 7FFFh).

Example

The following command string:   0034h   0004h   7FFFh   7FFFh   FFFFh
will set the master output volume to maximum.

WARNING:  Aria systems without the SC18075 Mixer chip cannot process this command.  DO NOT send this command unless you are sure of the Aria hardware configuration.

Tone ControlXE "COMMANDS:Tone Control"§XE "Tone Control"§	Command ID 35h

This command sets the bass and treble tone controls for the mixer output.

| 0035 | xxyy | xxyy | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

left	xx	Left channel treble control
		yy	Left channel bass control

right	xx	Right channel treble control
		yy	Right channel bass control

All control parameters listed above (xx or yy) are byte values and can range from:

0	Flat response
1 to 7	Minimum frequency cut to maximum frequency cut

8	Flat response
9 to 15	Minimum frequency boost to maximum frequency boost

WARNING:  Aria systems without the SC18075 Mixer chip cannot process this command.  DO NOT send this command unless you are sure of the Aria hardware configuration.

### XE "Mixer Commands" \r "MixerCommands"§

### 2.5.5	Audio Effect Commands

This section describes all commands used to control the reverb and QSound Virtual Audio effects that are applied to the output of the Aria Synthesizer and digital audio channels.

NOTE:  Systems without reverb or QSound loaded CANNOT ACCEPT these commands.

Included are the following commands:

| Command ID 72h 75h 76h 77h 80h | Command Name Reverb Master Level Reverb Mode Reverb Digital Audio Level Reverb WPT Level QSound Virtual Audio Mode |
| --- | --- |

Reverb Master LevelXE "COMMANDS:Reverb Master Level"§XE "Reverb"§	Command ID 72h

This command sets the overall reverb output level.

| 0072 | xxxx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | left | right |  |

left	Left channel reverb level (0 to 7FFFh).

right	Right channel reverb level (0 to 7FFFh).

WARNING:  Aria systems that have not loaded the 
reverb DSP application cannot process this command.
DO NOT send this command unless you are sure of the 
specific Aria configuration.

Reverb ModeXE "COMMANDS:Reverb Mode"§	Command ID 75h

This command sets the reverb effect mode.

| 0075 | 000x | FFFF |
| --- | --- | --- |
| ID | mode |  |

mode	0	Room 1
1	Room 2
2	Room 3
3	Hall 1
4	Hall 2
5	Plate Echo
6	Delay

WARNING:  Aria systems that have not loaded the 
reverb DSP application cannot process this command.
DO NOT send this command unless you are sure of the 
specific Aria configuration.

Reverb Digital Audio LevelXE "COMMANDS:Reverb Digital Audio Level"§XE "Reverb"§	Command ID 76h

This command sets the reverb level for all digital audio output channels.

| 0076 | xxxx | FFFF |
| --- | --- | --- |
| ID | level |  |

level	Digital audio reverb level (0 to 7FFFh).

WARNING:  Aria systems that have not loaded the 
reverb DSP application cannot process this command.
DO NOT send this command unless you are sure of the 
specific Aria configuration.

Reverb WPT LevelXE "COMMANDS:Reverb WPT Level"§	Command ID 77h

This command sets the reverb level for a specific Aria Synthesizer wave parameter table.  This can be used to control the reverb for individual MIDI channels.

| 0077 | 00xx | xxxx | FFFF |
| --- | --- | --- | --- |
| ID | WPT # | level |  |

WPT #	Wave Parameter Table number (0 to 4Fh).

level	Reverb level (0 to 7FFFh).

WARNING:  Aria systems that have not loaded the 
reverb DSP application cannot process this command.
DO NOT send this command unless you are sure of the 
specific Aria configuration.

QSound Virtual Audio ModeXE "COMMANDS:QSound Mode"§	XE "QSound"§Command ID 80h

This command sets the QSound Virtual Audio effects mode.

| 0080 | 000x | FFFF |
| --- | --- | --- |
| ID | mode |  |

mode	0	Off
1	Apply effect to digital audio channels only
2	Apply effect to Aria Synthesizer output only
3	Apply effect to both digital audio and Aria Synthesizer

WARNING:  Aria systems that have not loaded the 
QSound DSP application cannot process this command.
DO NOT send this command unless you are sure of the 
specific Aria configuration.

XE "Communication Interface" \r "Communication"§XE "COMMANDS" \r "Commands"§

< This page intentionally left blank >