E2000plus-series Ethernet Adapters

Installation file for

\NDIS directory

============================================================================

**********************************************

*  Driver Installation for LAN Manager v2.1  *

**********************************************

1. Insert the E2000plus driver diskette into drive A and a formatted

blank diskette into drive B.

2. Invoke A:\NDIS\LM.BAT to transfer files from Drive A to B.

3. Select "Other Driver" when invoking C:\LANMAN\SETUP.EXE to

install the NDIS driver, and specify "B:" as the drive.

4. Check the PROTOCOL.INI file to make sure that the INTERRUPT

and IOBASE agree with the settings on the LAN card.

*********************************************

*  Driver Installation for LAN Server v3.0  *

*********************************************

With the E2000plus driver diskette in drive A:

1. Copy A:\NDIS\E2000.OS2 to C:\IBMCOM\MACS

2. Copy A:\NDIS\LS.NIF to C:\IBMCOM\MACS\E2000.NIF

3. Invoke C:\IBMCOM\LAPS.EXE to install the network driver

*******************************************************

*  Driver Installation for LAN Support Program v1.31  *

*******************************************************

With the E2000plus driver diskette in drive A:

1. Copy A:\NDIS\E2000.DOS to a LAN Support Program directory.

2. Add DEVICE=\PATH\E2000.DOS to the CONFIG.SYS file.

3. Modify the PROTOCOL.INI file under the LAN Support Program

directory as follows:

BINDINGS = E2000

:

:

[E2000]

DRIVERNAME = E2000$

IOBASE = 0X300

INTERRUPT = 3

***************************************************

*  NDIS 2.0.1 DOS Driver Installation for PC/TCP  *

***************************************************

With the E2000plus driver diskette in drive A:

1. Copy A:\NDIS\E2000.DOS into C:\PCTCP

(where PC/TCP for DOS has been installed).

2. Add the following line to the CONFIG.SYS file:

DEVICE=C:\PCTCP\E2000.DOS

For example,

DEVICE=C:\PCTCP\PROTMAN.DOS /I:C:\PCTCP

DEVICE=C:\PCTCP\E2000.DOS

DEVICE=C:\PCTCP\DIS_PKT.GUP

3. Edit the C:\PCTCP\ETC\PROTOCOL.INI file:

For example,

;++++++++++++++++++++++++

;++   ADAPTERS         ++

;++++++++++++++++++++++++

;  E2000 Series of cards

;

[E2000_NIF]

drivername = E2000$

interrupt = 3

iobase = 0x300

;++++++++++++++++++++++++

;++    PROTOCOLS       ++

;++++++++++++++++++++++++

;FTP PC/TCP for DOS

[PKTDRV]

drivername = PKTDRV$

bindings = E2000_NIF

intvec = 0x60

****************************************************

*  NDIS 2.0.1 OS/2 Driver Installation for PC/TCP  *

****************************************************

With the E2000plus driver diskette in drive A:

1. Copy A:\NDIS\E2000.OS2 to C:\PCTCP\DRV

2. Add the following line to the CONFIG.SYS file:

DEVICE=C:\PCTCP\DRV\E2000.OS2

For example,

DEVICE=C:\PCTCP\DRV\PROTMAN.OS2 /I:C:\PCTCP\DRV

DEVICE=C:\PCTCP\DRV\E2000.OS2

DEVICE=C:\PCTCP\DRV\SOCKET.OS2

3. Edit the C:\PCTCP\DRV\PROTOCOL.INI file:

For example,

;++++++++++++++++++++++++

;++   ADAPTERS         ++

;++++++++++++++++++++++++

;  E2000 Series of cards

;

[E2000_NIF]

DRIVERNAME = E2000$

INTERRUPT = 3

IOBASE = 0x300

;++++++++++++++++++++++++

;++     PROTOCOLS      ++

;++++++++++++++++++++++++

;FTP PC/TCP for OS/2

[SOCKET]

drivername = $SOCKET

bindings = E2000_NIF

*********************************************************

*  Driver Installation for Windows for Workgroups v3.1  *

*********************************************************

With the E2000plus driver diskette in drive A, start WFW 3.1

and click on the Network icon in the Control Panel.

1. If there is no E2000 NDIS driver installed on your hard

disk, you need to select the "Unlisted or Updated Network

Adapter" and specify "A:\NDIS" as the path.

Skip to Step 3.

2. Select "E2000 Series Ethernet Adapters" in the network

adapter list.

3. Specify the correct configurations of NIC, including I/O Base

and INTERRUPT.

4. Exit and reboot the PC to put the change into effect.

NOTE: If you previously installed the driver in WFW and you want to install

it from the diskette again, you must delete the related OEM?.INF file

in the :\WINDOWS\SYSTEM subdirectory.

************************************************

*  Driver Installation for Banyan VINES v4.10  *

************************************************

With the E2000plus driver diskette in drive A:

1. Copy A;\NDIS\E2000.DOS to the VINES directory that contains

the file BAN.EXE.

2. Create a LANMAN subdirectory under the root, and copy the

PROTOCOL.INI file to this directory.

3. Add the following two lines to your CONFIG.SYS file:

DEVICE=<VINES directory>\PROTMAN.DOS

DEVICE=<VINES directory>\E2000.DOS

4. Check the PROTOCOL.INI file to make sure that the INTERRUPT and

IOBASE agree with the settings on the LAN card.

5. Run PCCONFIG and configure the NDIS Ethernet driver.  Enter

the interrupt level and fill Section Name "E2000_NIF" in the

"BINDINGS =" field.

NOTE: If the version of Banyan Vines software is over 4.10, please

refer to Vines LAN driver for more detail.

************************************

*  Driver Installation for PC-NFS  *

************************************

With the E2000plus driver diskette in drive A:

1. Prepare the "QuickNDIS with drivers for PC-NFS" diskette and insert

it into floppy diskette drive A: or B:.

2. Run QUICKNDIS.

3. After the "NDIS Driver" menu appears, choose "zero" for the third

party driver.

4. Next, input path and make sure the driver, filename with extension NIF,

and PROTOCOL.INI are in the diskette drive with path "NDIS".

5. Confirm whether the driver name is correct by "E2000 Series Ethernet

Adapters" message.

6. When the driver is installed, you are required to configure the

IRQ, I/O address, and driver name. Default values are 0x300, 3, and

E2000$.

7. QUICKNDIS program will update the CONFIG.SYS and AUTOEXEC.BAT

files automatically.  After these operations, reboot the workstation.

The network card driver will be loaded into memory.