E2000plus-series Ethernet Adapters

Installation file for

\NETWARE directory

============================================================================

*****************************************************

*  Installation for NetWare IPX Workstation Driver  *

*****************************************************

The NetWare IPX workstation driver file is stored in the \NETWARE\IPX

subdirectory.

1. At DOS prompt '>', type the following command to execute IPX workstation

driver.

[drive]:\NETWARE\IPX\IPX.COM

2. After executing the driver, run NETX.EXE

3. The following example is to change the IPX setting according to hardware

configuration.

a) Display help screen.

[drive]:\NETWARE\IPX\IPX.COM /?

Novell IPX/SPX  v3.10 (911121)

(C) Copyright 1985, 1991 Novell Inc.  All Rights Reserved.

LAN Option: NE2000 Compatible Ethernet  v2.10 (950227)

Hardware Configuration: Auto Configuration

Usage: IPX [options]

valid options:

-I or /I                  Display version information

-D or /D                  Display hardware options

-O or /O<num>             Load using hardware option <num>

-C or /C=[path]<filename> Use an alternate configuration file

-? or /?                  Display this help screen

b) Display hardware options.

[drive]:\NETWARE\IPX\IPX.COM /D

Novell IPX/SPX  v3.10 (911121)

(C) Copyright 1985, 1991 Novell Inc.  All Rights Reserved.

LAN Option: NE2000 Compatible Ethernet  v2.10 (950227)

Hardware options available:

*  0. Auto Configuration

1. IRQ = 3,      I/O Base = 300H, no DMA or RAM

2. IRQ = 2 or 9, I/O Base = 320H, no DMA or RAM

3. IRQ = 4,      I/O Base = 340H, no DMA or RAM

4. IRQ = 5,      I/O Base = 360H, no DMA or RAM

5. IRQ = 2 or 9, I/O Base = 300H, no DMA or RAM

6. IRQ = 3,      I/O Base = 240H, no DMA or RAM

7. IRQ = 5,      I/O Base = 280H, no DMA or RAM

8. IRQ = 4,      I/O Base = 2C0H, no DMA or RAM

9. IRQ = 4,      I/O Base = 300H, no DMA or RAM

10. IRQ = 5,      I/O Base = 320H, no DMA or RAM

11. IRQ =15,      I/O Base = 340H, no DMA or RAM

12. IRQ =15,      I/O Base = 360H, no DMA or RAM

13. IRQ = 5,      I/O Base = 300H, no DMA or RAM

14. IRQ =10,      I/O Base = 320H, no DMA or RAM

15. IRQ =11,      I/O Base = 340H, no DMA or RAM

16. IRQ =12,      I/O Base = 360H, no DMA or RAM

c) Change IPX hardware options.

For example:

[drive]:\NETWARE\IPX\IPX.COM /O4

*********************************************************

*  Installation for NetWare DOS ODI Workstation Driver  *

*********************************************************

The NetWare DOS ODI workstation driver file is stored in the \NETWARE\DOSODI

subdirectory.

1. Power on the workstation, and execute Novell's LSL program.

2. At the DOS prompt type the following command to execute the NetWare 4.x

workstation driver.

[drive]:\NETWARE\DOSODI\E2000

where [drive] is the floppy disk drive that this diskette is in.

3. After executing the driver, run NetWare's IPXODI.COM & NETX.EXE

(or VLM.EXE) files.

4. Please refer to Novell's Installation Manual for more details.

**************************************************

*  Driver Installation for Personal NetWare 1.x  *

**************************************************

The driver for Personal NetWare 1.x is stored in the \NETWARE\DOSODI subdirectory.

1. First install Personal NetWare in the PC.

2. Copy the driver file to the Personal NetWare subdirectory.

At the DOS prompt, type the following commands:

COPY A:\NETWARE\DOSODI\E2000.INS C:\NWCLIENT

COPY A:\NETWARE\DOSODI\E2000.COM C:\NWCLIENT

NOTE: This assumes the diskette is in Drive A:.

3.  a) Run Personal NetWare's SETUP.EXE to enter the main menu.

b) Select "Primary Network Interface Card" item, and press ENTER to

select "E2000 Ethernet Card".

c) Return to the main menu to select "Configure Primary Interface Card"

to configure the "INT", "PORT" and "Media Frame Type(s)".

d) At the main menu, press "Save Changes and Exit".

4. Execute STARTNET.BAT.

***************************************************

*  Installation for Novell LAN WorkPlace for DOS  *

***************************************************

1. First, install Novell LAN WorkPlace.

2. Use a text editor to modify C:LANWP.BAT as follows:

C:\yesno "Do you want to load the networking software? [y/n] "

if errorlevel 1 goto noload

PATH C:\LANWP4\BIN;%path%

C:\lsl.com

>> C:\ne2000.com  << ---  modify NE2000.COM to E2000.COM

C:\ipxodi.com

C:\netx.com

...

...

break on

:noload

3. Next, use the text editor to modify C:NET.CFG like this:

>> Link Driver NE2000   << --- modify NE2000 to E2000

Protocol IPX 0 Ethernet_802.3

Frame Ethernet_802.3

Frame Ethernet_II

PORT    300

INT     3

...

...

4. At the DOS prompt, type the following command:

Copy A:\NETWARE\DOSODI\E2000.COM C:\

NOTE: This assumes that this diskette is in the "A:" disk drive.

5. Run LANWP.BAT

**********************************************************

*  Installation for NetWare OS/2 ODI Workstation Driver  *

**********************************************************

The NetWare ODI driver for OS/2 is stored in the \NETWARE\OS2ODI subdirectory.

1. First, install NetWare Requester for OS/2.

2. Copy E2000 driver to the subdirectory of NetWare.

At the OS/2 prompt, type the following commands:

COPY A:\NETWARE\OS2ODI\E2000.SYS C:\NETWARE\

NOTE: This assumes the diskette is in Drive A:.

3. Use a text editor to modify C:\CONFIG.SYS as follows:

...

...

DEVICE=C:\NETWARE\LSL.SYS

RUN=C:\NETWARE\DDAEMON.EXE

>> DEVICE=C:\NETWARE\NE2000.SYS << ---  modify NE2000.SYS to E2000.SYS

...

...

4. Use a text editor to create C:\NETWARE\NET.CFG.

The following is an example of the NET.CFG file:

Link driver E2000

INT 3

PORT 300

FRAME Ethernet_802.3

5. Shut down and reboot the computer.

NOTE: There is a text editor named E.EXE to use under OS/2 full screen.

To edit the two files above, at the OS/2 prompt, type:

E C:\CONFIG.SYS

or

E C:\NETWARE\NET.CFG

******************************************************

*  Installation for NetWare 386 v3.11 Server Driver  *

******************************************************

The NetWare 3.11 ODI server driver file is stored in the \NETWARE\SERVER.31X

subdirectory.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load LSLENH.NLM first (this also automatically loads PATCHMAN.NLM).

At the server's prompt, type:

LOAD A:\NETWARE\SERVER.31X\LSLENH.NLM

3. Load LAN driver E2000.LAN and bind it. This will automatically load

MSM31X.NLM and ETHERTSM.NLM. The general way of using these parameters

is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NETWARE\SERVER.31X\E2000.LAN PORT=300 INT=3 FRAME=Ethernet_802.2 NAME=E2000_802.2

BIND IPX TO E2000_802.2 NET=2

LOAD A:\NETWARE\SERVER.31X\E2000.LAN PORT=300 INT=3 FRAME=Ethernet_802.3 NAME=E2000_802.3

BIND IPX TO E2000_802.3 NET=3

NOTE: This assumes that this diskette is in drive "A:".

4. The server program will load and execute the E2000.LAN driver module.

Please refer to the NetWare 386 V3.11 Installation Manual for more

information to make an auto-execution batch file to load the E2000.LAN

module.

NOTE: If you use the MONITOR.NLM shipped with NetWare 3.11, your server could

abend when viewing LAN statistics in MONITOR.NLM with E2000.LAN driver.

We have provided the newer Novell MONITOR.NLM program in the driver

diskette to correct this problem.

******************************************************

*  Installation for NetWare 386 v3.12 Server Driver  *

******************************************************

The NetWare 3.12 ODI server driver file is stored in the \NETWARE\SERVER.31X

subdirectory.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load the LAN driver E2000.LAN and bind it (this also automatically loads

MSM31X.NLM and ETHERTSM.NLM). The general way of using these parameters

is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NETWARE\SERVER.31X\E2000.LAN PORT=300 INT=3 FRAME=Ethernet_802.2 NAME=E2000_802.2

BIND IPX TO E2000_802.2 NET=2

LOAD A:\NETWARE\SERVER.31X\E2000.LAN PORT=300 INT=3 FRAME=Ethernet_802.3 NAME=E2000_802.3

BIND IPX TO E2000_802.3 NET=3

NOTE: This assumes that this diskette is in drive A.

3. The server program will load and execute the E2000.LAN driver module.

Please refer to the NetWare 3.12 Installation Manual for more

information on how to make an auto-execution batch file to load the

E2000.LAN module.

******************************************************

*  Installation for NetWare v4.x  Server LAN Driver  *

******************************************************

The NetWare 4.x ODI server driver file is stored in the \NETWARE\SERVER.4

subdirectory.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load the LAN driver E2000.LAN. (Two different methods are shown here.)

METHOD 1 --

At the server's prompt, load the driver E2000.LAN and bind it

(this also automatically loads MSM.NLM and ETHERTSM.NLM). The general

way of using these parameters is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NETWARE\SERVER.4\E2000.LAN PORT=300 INT=3 FRAME=Ethernet_802.2 NAME=E2000_802.2

BIND IPX TO E2000_802.2 NET=2

LOAD A:\NETWARE\SERVER.4\E2000.LAN PORT=300 INT=3 FRAME=Ethernet_802.3 NAME=E2000_802.3

BIND IPX TO E2000_802.3 NET=3

NOTE: This assumes that this diskette is in drive A.

METHOD 2 --

a) At the server's prompt type: LOAD INSTALL

b) When the install menu appears, choose the menu item.

c) Install the LAN driver E2000.LAN according to the NetWare manual,

or use the on-line instructions.

3. The server program will load and execute the E2000.LAN driver module.

Please refer to the NetWare 4.x Installation Manual for more information

on making an auto-execution batch file to load the E2000.LAN module.