Club 100 Library - 415/939-1246 BBS,    937-5039 NEWSLETTER, 932-8856 VOICE

How the Linefeed Utilities Work

-------------------------------

This is how the print linefeed patch in FIXLF/LFUTL (and LFEED.200)

works.  The string poked into the F8 key memory location in high

RAM installs a machine language program into the definition for

this key:

dec   hex      code

---   ---   -----------

254    FE

13    0D   CPI  0Ah   ;is it a CR?

245    F5   PUSH AF    ;save accum on stack

62    3E

10    0A   MOV  A,0Ah ;put LF in accum

204    CC

63    3F

109    6D   CZ   6D3Fh ;ROM call to print LF if CR was found

241    F1   POP  AF    ;restore the accum char at entry

201    C9   RET        ;return to normal print routine

This program is entered from the print routine in high RAM, if the

patch defined by A (in FIXLF/LFUTL) is poked in to cause the jump;

the pokes make the print routine jump to the F8 area where this

program resides.

All M100 print LF patches appear to work in this manner.  It's all

a matter of where the code is installed.  In a print processor

such as LAPWRD it can be installed within the program.  In

LPTLF.100 it is placed just under MAXRAM, a disaster if other m/l

programs want that space.  It is also possible (recent DL4 file)

to put it into the bottom of RAM (first Basic program); but there

it can conflict with SUPERA and/or the PG Designs MENU utility.

The function key location is surprisingly safe.  However, it will

be zapped by any Basic program which has the following code (M100

manual, p.149):

CALL 23164,0,23366

CALL 27795

or M/L programs with equivalent code.  A common offender is the

original WRITE+.BA from PCSG (aka SCRIPTSIT, more or less) --

which reset the keys (without ever having changed them, so this

superfluous code can be removed).  Hugo Ferreya wrote FIXLF.HF1 (I

think in response to a message of mine) and uploaded it here.

I've used it for about two years with no problems.

Phil Wheeler

The program LFUTL.BA has caused problems for some SIG members who

use M/L programs.  To eliminate this difficulty (caused by a

'CLEAR256,MAXRAM' statement), I suggest all past and future users

change line 8 to read: 8 MENU, before running the program. This

change will eliminate clearing high user RAM and will also avoid

resetting your clock to 1984 (ok now, but not for long).

Phil Wheeler:11/12/84

3F

109    6D   CZ   6D3Fh ;ROM call to print LF if CR was found

241    F1   POP  AF    ;restore the accum char at entry

201    C9   RET        ;return to normal print routine

This program is entered from the print routine in high RAM, if the

patch defined by A (in FIXLF/LFUTL) is poked in to cause the jump;

the pokes make the print routine jump to the F8 area where this

program resides.

All M100 print LF patches appear to work in this manner.  It's all

a matter of where the code is installed.  In a print processor

such as LAPWRD it can be installed within the program.  In

LPTLF.100 it is placed just under MAXRAM, a disaster if other m/l

programs want that space.  It is also possible (recent DL4 file)

to put it into the bottom of RAM (first Basic program); but there

it can conflict with SUPERA and/or the PG Designs MENU utility.

The function key location is surprisingly safe.  However, it will

be zapped by any Basic program which has the following code (M100

manual, p.149):

CALL 23164,0,23366

CALL 27795

or M/L programs with equivalent code.  A common offender is the

original WRITE+.BA from PCSG (aka SCRIPTSIT, more or less) --

which reset the keys (without ever having changed them, so this

superfluous code can be removed).  Hugo Ferreya wrote FIXLF.HF1 (I

think in response to a message of mine) and uploaded it here.

I've used it for about two years with no problems.

Phil Wheeler