Here's an excerpt from The Modem Reference, written by Michael A.

Banks and recommended by Jerry Pournelle in Byte, The Smithsonian

Magazine, et al.

The right to reproduce this article is granted on the condition

that all text, including this notice and the notice at the end of

the article, remain unchanged, and that no text is added to the

body of the article.

Thanks!  --MB

BITS, BAUD RATE, AND BPS

Taking the Mystery Out of Modem Speeds

by Michael A. Banks

(Copyright, 1988, Michael A. Banks.  All rights reserved.)

Modem transmission speed is the source of a lot of

confusion, even among otherwise informed computer and modem

users.  The root of the problem is the fact that the terms "baud"

and "bits per second" are used interchangeably and

indiscriminately.  I strongly suspect this is a result of the

fact that it's easier to say "baud" than "bits per second,"

though misinformation has a hand in it, too.

If you've ever found yourself confused by the relationship

between bits and baud rate, or if you think that a modem's baud

rate is the same as the number of bits or characters it transmits

per second, please read this article carefully; I guarantee to

clear up the confusion and disabuse you of any false concepts ...

Bits per second (bps)

Bits per second is a measure of the number of data bits

(digital 0's and 1's) transmitted each second in a communications

channel.  This is sometimes referred to as "bit rate."

Individual characters (letters, numbers, etc.), also

referred to as bytes, are composed of several bits.

While a modem's bit rate is tied to its baud rate, the two

are not the same, as explained below.

Baud rate

Baud rate is a measure of the number of times per second a

signal in a communications channel varies, or makes a transition

between states (states being frequencies, voltage levels, or

phase angles).  One baud is one such change.  Thus, a 300-baud

modem's signal changes state 300 times each second, while a 600-

baud modem's signal changes state 600 times per second.  This

does not necessarily mean that a 300-baud and a 600-baud modem

transmit 300 and 600 bits per second, as you'll learn in a few

lines.

Determining bits per second

Depending on the modulation technique used, a modem can

transmit one bit--or more or less than one bit--with each baud,

or change in state.  Or, to put it another way, one change of

state can transmit one bit--or more or less than one bit.

As I mentioned earlier, the number of bits a modem transmits

per second is directly related to the number of bauds that occur

each second, but the numbers are not necessarily the same.

To illustrate this, first consider a modem with a baud rate

of 300, using a transmission technique called FSK (Frequency

Shift Keying, in which four different frequencies are turned on

and off to represent digital 0 and 1 signals from both modems).

When FSK is used, each baud (which is, a gain, a change in state)

transmits one bit; only one change in state is required to send a

bit.  Thus, the modem's bps rate is also 300:

300 bauds per second X 1 bit per baud  =  300 bps

Similarly, if a modem operating at 1200 baud were to use one

change in state to send each bit, that modem's bps rate would be

1200.  (There are no 1200 baud modems, by the way; remember that.

This is only a demonstrative and hypothetical example.)

Now, consider a hypothetical 300-baud modem using a

modulation technique that requires two changes in state to send

one bit, which can also be viewed as 1/2 bit per baud.  Such a

modem's bps rate would be 150 bps:

300 bauds per second X 1/2 baud per bit  =  150 bps

To look at it another way, bits per second can also be

obtained by dividing the modem's baud rate by the number of

changes in state, or bauds, required to send one bit:

300 baud

---------------  =  150 bps

2 bauds per bit

Now let's move away from the hypothetical and into reality,

as it exists in the world of modulation.

First, lest you be misled into thinking that "any 1200 baud

modem" should be able to operate at 2400 bps with a two-bits-per-

baud modulation technique, remember that I said there are no 1200

baud modems.  Medium- and high-speed modems use baud rates that

are lower than their bps rates.  Along with this, however, they

use multiple-state modulation to send more than one bit per baud.

For example, 1200 bps modems that conform to the Bell 212A

standard (which includes most 1200 bps modems used in the U.S.)

operate at 300 baud and use a modulation technique called phase

modulation that transmits four bits per baud.  Such modems are

capable of 1200 bps operation, but not 2400 bps because they are

not 1200 baud modems; they use a baud rate of 300.  So:

300 baud X 4 bits per baud  =  1200 bps

or

300 baud

------------------  =  1200 bps

1/4 baud per bit

Similarly, 2400 bps modems that conform to the CCITT V.22

recommendation (virtually all of them) actually use a baud rate

of 600 when they operate at 2400 bps.  However, they also use a

modulation technique that transmits four bits per baud:

600 baud X 4 bits per baud  =  2400 bps

or

600 baud

------------------  = 2400 bps

1/4 baud per bit

Thus, a 1200-bps modem is not a 1200-baud modem, nor is a

2400-bps modem a 2400-baud modem.

Now let's take a look at 9600-bps modems.  Most of these

operate at 2400 baud, but (again) use a modulation technique that

yields four bits per baud.  Thus:

2400 baud X 4 bits per baud  =  9600 bps

or

2400 baud

------------------  =  9600 bps

1/4 baud per bit

Characters per second (cps)

Characters per second is the number of characters (letters,

numbers, spaces, and symbols) transmitted over a communications

channel in one second.  Cps is often the bottom line in rating

data transmission speed, and a more convenient way of thinking

about data transfer than baud- or bit-rate.

Determining the number of characters transmitted per second

is easy: simply divide the bps rate by the number of bits per

character.  You must of course take into account the fact that

more than just the bits that make up the binary digit

representing a character are transmitted when a character is sent

from one system to another.  In fact, up to 10 bits may be

transmitted for each character during ASCII transfer, whether 7

or 8 data bits are used.  This is because what are called start-

and stop-bits are added to characters by a sending system to

enable the receiving system to determine which groups of bits

make up a character.  In addition, a system usually adds a parity

bit during 7-bit ASCII transmission.  (The computer's serial port

handles the addition of the extra bits, and all extra bits are

stripped out at the receiving end.)

So, in asynchronous data communication, the number of bits

per character is usually 10 (either 7 data bits, plus a parity

bit, plus a start bit and a stop bit, or 8 data bits plus a start

bit and a stop bit).  Thus:

300 bps

-----------------------  =  30 characters per second

10 bits per character

1200 bps

-----------------------  =  120 characters per second

10 bits per character

2400 bps

-----------------------  =  240 characters per second

10 bits per character

Common speeds

The most commonly-used communications rates for dial-up

systems (BBSs and online services like CompuServe, DELPHI, and

GEnie) are 300, 1200, and 2400 bps.  A few older systems--

especially Telex systems--communicate at 110 bps, but these are

gradually going the way of the dinosaur.  4800 and 9600 bps

modems are generally available, but few online services or BBSs

accommodate them.  This will be changing in the near future,

however, with the cost of high-speed modem technology decreasing

as the demand for it increases.

Modems with even higher bps rates are manufactured (19,200

and up) but these are not used with dial-up systems; the upper

limit on asynchronous data transmission via voice-grade telephone

lines appears to be 9600 bps.  The use of higher transmission

rates requires special dedicated lines that are "conditioned"

(i.e., shielded from outside interference) as well as expensive

modulation and transmission equipment.

If you found this article useful, you may want to pick up a

copy of the book from which it was excerpted:

THE MODEM REFERENCE

by Michael A. Banks

Published by Brady Books/Simon & Schuster

ISBN # 0-13-586646-4    $19.95

In addition to explaining the technical aspects of modem

operation, communications software, data links, and other

elements of computer communications, the book provides detailed,

illustrated "tours" of major online services such as UNISON,

CompuServe, DELPHI, BIX, Dow Jones News/Retrieval, MCI Mail, and

others.  It also contains information on using packet switching

networks and BBSs, as well as dial-up numbers for various

networks and BBSs.

You'll also find hands-on guides to buying, setting up,

using, and troubleshooting computer communications hardware and

software.  (And the book "supports" all major microcomputer

brands.)

Other books by Michael A. Banks: GETTING THE MOST OUT OF

DESKMATE 3 (Brady Books/Simon & Schuster, available at your local

Radio Shack store); QUICK & EASY GUIDE TO REFLEX 2 (Compute!

Books); UNDERSTANDING FAX & E-MAIL (Howard W. Sams & Co.); WORD

PROCESSING SECRETS FOR WRITERS (Writer's Digest Books); THE

ODYSSEUS SOLUTION (SF novel; Baen Books).

For more information, contact:

Michael A. Banks

P.O. Box 312

Milford, OH  45150